This market manual is provided for stakeholder engagement purposes.  Proposed changes, to be effective for the December 2022 capacity auction, are indicated based on the current version of the market manual.  Please note that additional changes to this document may be incorporated as part of future engagement on design enhancements to the capacity auction or other IESO activities prior to this market manual taking effect.

PUBLIC

This market manual provides guidance to market participants on the operation of the capacity auction process

MAN-44

PUBLIC

Market Manual 12.0: Capacity Auctions

Issue 13.0

PUBLIC

This market manual provides guidance to market participants on the operation of the capacity auction process

MAN-44

PUBLIC

MANUAL

Disclaimer

The posting of documents on this Website is done for the convenience of market participants and other interested visitors to the IESO Website. Please be advised that, while the IESO attempts to have all posted documents conform to the original, changes can result from the original, including changes resulting from the programs used to format the documents for posting on the Website as well as from the programs used by the viewer to download and read the documents. The IESO makes no representation or warranty, express or implied that the documents on this Website are exact reproductions of the original documents listed. In addition, the documents and information posted on this Website are subject to change. The IESO may revise, withdraw or make final these materials at any time at its sole discretion without further notice. It is solely your responsibility to ensure that you are using up-to-date documents and information.

This market manual may contain a summary of a particular market rule. Where provided, the summary has been used because of the length of the market rule itself. The reader should be aware, however, that where a market rule is applicable, the obligation that needs to be met is as stated in the “Market Rules”. To the extent of any discrepancy or inconsistency between the provisions of a particular market rule and the summary, the provision of the market rule shall govern.

Document ID	MAN-44

Document Name	Market Manual 12.0: Capacity Auctions

Issue	Issue 13.0

Reason for Issue	Issue released for Baseline 46.0

Effective Date	September 15, 2021

Document ID	MAN-44

Document Name	Market Manual 12.0: Capacity Auctions

Issue	Issue 13.0

Reason for Issue	Issue released for Baseline 46.0

Effective Date	September 15, 2021

Document Change History

| Issue | Reason for Issue | Date |
| --- | --- | --- |
| 1.0 | Issue released in advance of Baseline 34.1 | September 30, 2015 |
| 2.0 | Issue released for Baseline 34.1 | December 2, 2015 |
| 3.0 | Issue released in advance of Baseline 35.1 | March 22, 2016 |
| 4.0 | Issue released for Baseline 36.1 | December 7, 2016 |
| 5.0 | Issue released in advance of Baseline 37.0 | January 11, 2017 |
| 6.0 | Issue released in advance of Baseline 37.1 | March 31, 2017 |
| 7.0 | Issue released in advance of Baseline 42.1 | October 15, 2019 |
| 8.0 | Issue released in advance of Baseline 43.1. These changes are applicable to the demand response auction held in December 2019, for the commitment period beginning May 1, 2020. | April 23, 2020 |
| 9.0 | Issue released in advance of Baseline 43.1. These changes are applicable to capacity auctions to be for commitment periods beginning on or after May 1, 2021. | May 4, 2020 |
| 10.0 | Issue released in advance of Baseline 44.0 | June 29, 2020 |
| 11.0 | Issue released for Baseline 44.0 | September 16, 2020 |
| 12.0 | Updated to meet accessibility requirements pursuant to the Accessibility for Ontarians with Disabilities Act | December 2, 2020 |
| 13.0 | Issue released for Baseline 46.0 | September 15, 2021 |

Table of Contents

Table of Contents	i

List of Figures	iii

List of Tables	iii

Table of Changes	iv

Market Manuals	1

1.	Introduction	2

1.1	Purpose	2

1.2	Scope	2

1.3	Who Should Use This Manual	3

1.4	Contact Information	3

1.5	Applicability	3

2.	Capacity Auction Overview	5

2.1	Capacity Auction Process	5

2.2	Capacity Auction Timelines	6

2.3	Commitment Periods and Obligation Periods	8

2.4	Availability Window	8

2.5	Demand Curve Elements	9

Target Capacity	10

Reference Price	10

Maximum and Minimum Auction Clearing Price	10

Capacity Limits	10

2.6	Zonal Constraints	11

2.7	Capacity Import Constraints	11

3.	Pre-Auction Requirements	12

3.1	Pre-Auction Report	12

3.2	Participant Authorization	13

3.3	Capacity Qualification	13

Submission of a Capacity Qualification Request	14

Capacity Qualification Assessment	16

3.4	Capacity Auction Deposit	18

4.	Auction Mechanics	21

4.1	Stage 1: Offer Submission and Validation	21

4.2	Stage 2: Auction Clearing	22

4.3	Stage 3: Post-Auction Reporting Obligations	23

5.	Post-Auction Requirements	25

5.1	Participant Authorization	25

Prudential Support	25

5.2	Registration Requirements	25

Contributor Management	27

5.3	Energy Market Participation	30

Outage Management/ Non-Performance Events	31

Measurement Data Submissions	31

Testing of Capacity Auction Resources	36

5.4	Measurement Data Audit for Virtual HDR Resources	48

Capacity Market Participant’s Responsibilities	48

Virtual C&I HDR Resource Audit	48

Audit Scheduling and Submission of Supporting Documents	49

Procedure to Conduct a Virtual C&I HDR Audit	50

Audit Review and Remedial Actions	51

Closure of Audit	51

6.	Settlements	52

6.1	Non-Performance Factors	52

7.	Buy-out Process	54

8.	Capacity Obligation Transfer	56

Appendix A: Template for DR Residential Contributor Management Registration	58

Appendix B: Template for Measurement Data Control Sheet	59

References	60

List of Figures

Figure 2-1: Capacity Auction Process	4

Figure 2-2: Capacity Auction Timeline	6

Figure 2-3: Periods of time related to capacity auctions	7

Figure 2-4: Downward Sloping Demand Curve	8

Figure 4-1: Capacity Auction Mechanics Overview	16

Figure 4-2: Auction Selection Process with Zonal Limits	17

Figure 5-1: Randomized Control Trials (RC) performance evaluation	25

Figure 5-2: Sample CSV File Format for Measurement Data Submission for C&I HDR	27

Figure 5-3: Sample CSV File Format for Measurement Data Submission for Residential HDR	28

List of Tables

Table 6-1: Non-Performance Factors	39

Table of Changes

| Reference (Section and Paragraph) | Description of Change |
| --- | --- |
| Section 3.2 | Wording included to clarify that the Capacity Auction Participant (CAP) will become the Capacity Market Participant (CMP) to ensure participants are aware that ownership requirements for physical resources will apply to CAPs as well. Inclusion of “Electricity Storage Participant” to list of classes of participation for which a CAP may also authorize. |
| Section 4.1 | Wording included to clarify that a CAP may revise a capacity auction offer in Online IESO up until the capacity auction offer window closes. |
| Section 5.2 | Wording included to clarify that CAP, except in the case of imports, must be the registered owner of the resource as defined in Market Manual 1.5. Wording included to clarify that a CAP with a physical capacity obligation providing demand response (DR) must register the DR resource as hourly demand response (HDR) or dispatchable load. Wording included to clarify a CAP with virtual HDR resource may include physical non-dispatchable load as DR contributors. Wording amended to clarify timelines for registration activities. |
| Section 5.3.3 | Remove dispatchable load testing instruction that applies to obligation period ending April 2021 (2019 DR Auction). |
| Section 5.6 | Wording amended to reflect non-performance charges occur if measurement data submission was not deemed accurate, timely or complete (to align with Market Rules) and for failing to deliver capacity in the energy market in a test activation. |
| Section 8 | Wording amended to clarify a capacity obligation transfer will be valid for all of the associated obligation period. Wording added to clarify that if a CAP or a CMP wants to submit multiple transfer partial requests from the same capability obligation, the IESO must approve the first request before the second request can be submitted to the IESO. |
| Throughout | Defined terms have replaced previously undefined terms (e.g., contributors vs demand response contributors). Defined terms are italicized. |

Market Manuals

The market manuals consolidate the market procedures and associated forms, standards, and policies that define certain elements relating to the operation of the IESO-administered markets. Market procedures provide more detailed descriptions of the requirements for various activities than is specified in the “Market Rules”. Where there is a discrepancy between the requirements in a document within a market manual and the “Market Rules”, the “Market Rules” shall prevail. Standards and policies appended to, or referenced in, these procedures provide a supporting framework.

– End of Section –

# Introduction

## Purpose

The IESO will conduct capacity auctions for the purpose of acquiring auction capacity through a competitive auction process (Ch. 7, S.18.1 of the market rules). The “Capacity Auctions” market manual is designed to provide market participants with an introduction to the capacity auction, operated by the IESO for the IESO-administered markets and the specific steps to be followed to conduct the auction. The market manual also provides information on market participants’ eligibility criteria, auction timelines, energy market participation, and settlement process.

## Scope

Capacity auctions, with respect to IESO-administered markets, are comprised of the following aspects:

- Market participant authorization as a capacity auction participant,

- Submission of a capacity qualification request by a capacity auction participant, determination of the amount of unforced capacity (UCAP) for each capacity auction resource by the IESO, and the submission of a capacity auction deposit by a capacity auction participant, ahead of an upcoming capacity auction,

- Submission of capacity auction offers by capacity auction participants,

- Processing of submitted capacity auction offers by the IESO and determining capacity obligations,

- Reporting of auction results and capacity obligations by the IESO,

- Energy market participation requirements,

- Testing of capacity auction resources, and

- Settlement process and capacity prudential support obligations.

In support of these aspects, this market manual details the conditions, actions, and timelines specific to the capacity auction by market participants and the IESO. This market manual also details the energy market participation requirements, settlement process, and capacity prudential support obligations for the capacity auctions by market participants and the IESO. The market manual is based on obligations expressed in the “Market Rules” (Ch. 2, Ch. 3, Ch. 7, and Ch. 9).

This market manual points to other market manuals and market rules that provide additional information.

## Who Should Use This Manual

The “Capacity Auctions” market manual is meant to be used by all those undertaking the following activities:

- Applicants seeking authorization as a capacity auction participant and/or capacity market participant for a capacity auction;

- Capacity auction participants seeking to qualify their capacity to determine their UCAP ahead of a capacity auction;

- Capacity auction participants seeking to submit capacity auction offers into the capacity auction;

- Capacity market participants seeking to register facilities in order to meet their capacity obligations through the energy market; and

- Capacity market participants seeking to satisfy a capacity obligation by participating in the energy market.

## Contact Information

Changes to this market manual are managed via the IESO Change Management process, which can be found on the Change Management Overview page (http://www.ieso.ca/sector-participants/change-management/overview). Stakeholders are encouraged to participate in the evolution of this market manual via this process.

To contact the IESO, you can email IESO Customer Relations at customer.relations@ieso.ca or use telephone or mail. Telephone numbers and the mailing address can be found on the Contact Page  (http://www.ieso.ca/corporate-ieso/contact). IESO Customer Relations staff will respond as soon as possible.

## Applicability

Pursuant to market rules Chapter 7, section 18.1A, a market participant that participates in a capacity auction will, until the end of that capacity auction’s commitment period, remain subject to those market manual provisions and corresponding market rules that were most recently in effect at the time of the holding of that capacity auction (except as provided by section 18.1A.3 in regards to urgent amendments). The versions of the market manuals in effect at the start of the capacity auction offer submission window specify the rights and obligations related to participation, satisfaction of capacity obligations, and performance of other requirements directly related to participation (notwithstanding any amendments that may have been made subsequent to the relevant capacity auction).

market participants consulting this market manual must verify that they are consulting the version of the market manual corresponding to the capacity auction in which they participated or wish to participate.

An archive of market manuals corresponding to particular capacity auctions, organized by date, can be found on the Capacity Auction Rules Library (https://www.ieso.ca/en/Sector-Participants/Change-Management/Capacity-Auction-Rules-Library).

– End of Section –

# Capacity Auction Overview

Capacity auctions acquire auction capacity for one commitment period, which consists of two obligation periods, referred to as summer and winter periods.

## Capacity Auction Process

Figure 2-1 below shows a representative capacity auction process overview:

Figure 2-1: Capacity Auction Process

## Capacity Auction Timelines

Ontario’s capacity auction follows the following timelines:

- The IESO will publish a pre-auction report prior to the start of the capacity qualification process for the capacity auction.

- Market participants intending to participate in the capacity auction must complete their authorization as a capacity auction participant in advance of initiating a capacity qualification request, and no later than 5 business days prior to the start of the qualification assessment window as specified in the pre-auction report

- During the qualification request submission window, capacity auction participants  are required to  submit a capacity qualification request identifying each potential capacity auction resource, and its corresponding installed capacity (ICAP) that they wish to have qualified in order to offer into the capacity auction.

- The IESO will complete an assessment to determine the unforced capacity (UCAP) for each potential capacity auction resource, and will notify capacity auction participants of this amount at the end of the qualification assessment window as specified in the pre-auction report.

- Capacity auction participants are required to submit the capacity auction deposit amount at least five business days prior to the start of the offer submission window for the capacity auction.

- The capacity auction will accept offers from capacity auction participants on the date announced in the pre-auction report, starting at 09:00 EST and ending on the next business day at 23:59 EST. This period is referred to as the offer submission window. Capacity auction participants intending to participate in the capacity auction must have submitted their capacity auction offers to the IESO during the offer submission window.

- The IESO will process all submitted capacity auction offers, determine clearing prices and quantities, and prepare and publish both the public and confidential post-auction reports by 16:00 EST, four business days following the day on which the offer submission window closes.

## Commitment Periods and Obligation Periods

The commitment period is the period of time for each capacity auction over which it secures auction capacity. It consists of two obligation periods, which are the periods of time for which a capacity market participant is required to satisfy its capacity obligation through the day-ahead commitment process and energy market.

There are two seasonal obligation periods for a capacity auction, defined as:

- Summer – May 1 to October 31

- Winter – November 1 to April 30

Forward period means the period of time following 3 business days after the auction period, to the commencement of an obligation period. The length of the forward period will vary depending on the date of a capacity auction relative to its obligation period.

Figure 2-3: Periods of time related to capacity auctions

Capacity auction participants may choose to submit capacity auction offers into either one or both of the obligation periods. The auction for both obligation periods requires separate capacity auction offers for each of the obligation periods. The two obligation periods will be evaluated individually using the submitted capacity auction offers compared to pre-determined demand curves, and will therefore have their own capacity auction clearing prices and quantities. Participants will receive a set of separate capacity obligations for each period, where applicable, if they successfully clear the auction. Capacity auction participants who secure a position in a capacity auction are required to complete their authorization and registration requirements, as applicable, during the forward period, as explained in Section 5 of this manual.

## Availability Window

(Market Rules: Ch. 7, ss. 19.4.1, 19.5.1, 19.7.1, 19.9.1, 19.9A.1 and 19.11.1)

The summer availability window will consist of business days from 12:00 to 21:00 EST (hour ending 13 to hour ending 21) and the winter availability window will consist of business days from 16:00 to 21:00 EST (hour ending 17 to hour ending 21).

All capacity market participants with a capacity obligation will receive an availability payment associated with their capacity obligation. Availability payments may be offset with non-performance charges in accordance with “Market Manual 5.5: Physical Markets Settlements Statements” during the associated obligation period.

## Demand Curve Elements

(Market Rules: Ch. 7, s. 18.5.2)

A capacity auction demand curve is a representation of the IESO’s willingness to acquire auction capacity; it defines the prices that the IESO is willing to pay for varying levels of auction capacity along the curve. The shape of the demand curve impacts the quantity (MW; the X-axis) and price ($/MW-day; the Y-axis) of auction capacity that will be acquired through an auction. The capacity auction uses a downward-sloping demand curve defined by the following parameters and illustrated in Figure 2-4 below:

- Target capacity

- Reference price

- Maximum and minimum auction clearing prices

- Capacity limits

Given the dynamic nature of the energy market, the IESO will review the demand curve parameters at least once every three years to ensure it is reflective of the current market conditions and system needs.

Figure 2-4: Downward Sloping Demand Curve

The key reference points on the downward-sloping curve shown above are further elaborated in the sections below.

### Target Capacity

The target capacity for each obligation period will be determined based on the reliability need or any additional need identified by the IESO. The target capacity for each obligation period shall be published by the IESO in the pre-auction report.

### Reference Price

The capacity auction reference price represents the price at which resources are incented to enter the market and recover the necessary costs to make their capacity available, recognizing their revenue opportunities and avoided costs in the energy market. The reference price is directly associated with the target capacity as another key reference point in the demand curve. (.

The capacity auction reference price for each obligation period shall be published by the IESO in the pre-auction report.

### Maximum and Minimum Auction Clearing Price

The maximum capacity auction clearing price is the maximum price that a capacity market participant may be paid for auction capacity. The maximum capacity auction clearing price is set at a multiple of 1.25 times the capacity auction reference price.

The minimum capacity auction clearing price is $0/MW-day.

The maximum and minimum capacity auction clearing price for each obligation period shall be published by the IESO in the pre-auction reports.

### Capacity Limits

The capacity limits used in the demand curve are:

- the minimum capacity,

- the maximum capacity at maximum capacity auction clearing price, and

- the maximum capacity

The minimum capacity is the minimum amount of auction capacity that the IESO will clear through a capacity auction for each obligation period.

The maximum capacity at maximum capacity auction clearing price will be determined based on the following formula:

MaxCap(MACP)=RP×TCMaxP

Where:

- MaxCap(MACP) is the maximum capacity at the maximum auction clearing price,

- RP is the capacity auction reference price,

- TC is the target capacity, and

- MaxP is the maximum capacity auction clearing price.

The maximum capacity is the maximum amount of auction capacity which the IESO will clear through the auction. The maximum capacity is determined by forming a straight line between the points defined by the maximum capacity at the maximum capacity auction clearing price and the target capacity at the capacity auction reference price, and extending this line to the price of $0/MW-day. The capacity limits for each obligation period shall be published by the IESO in the pre-auction report.

## Zonal Constraints

The ten electrical zones of Ontario are used to acquire auction capacity for the capacity auction. The IESO establishes zonal requirements or limits that will be used to set any minimum and maximum capacity limits, respectively, that can be cleared in the capacity auction for each electrical zone.

Each electrical zone has a set of capacity auction zonal constraints defined. These include

- minimum amount of auction capacity to be acquired

- total maximum amount of auction capacity that can be acquired

- maximum amount of auction capacity from resources not revenue-metered by the IESO (i.e. virtual resources) that can be acquired. This limit will not set the zonal capacity auction clearing price.

The capacity auction will establish an Ontario-wide capacity auction clearing price as well as possible zone specific capacity auction clearing prices. The IESO shall publish capacity auction zonal constraints in the pre-auction reports.

## Capacity Import Constraints

The external interfaces between the IESO-controlled grid and neighbouring systems may be used to acquire auction capacity for the capacity auction. The IESO will establish maximum capacity import limits that can be cleared in the capacity auction. These limits apply to system-backed capacity import resources and generator-backed capacity import resources.

Capacity auction offers associated with eligible system-backed capacity import resources and generator-backed capacity import resources will clear the capacity auction subject to the following constraints:

- maximum amount of auction capacity that may be provided by all  system-backed capacity import resources and generator-backed capacity import resources. When this constraint is binding it will not determine the capacity auction clearing price, and

- maximum amount of auction capacity that may be provided by all system-backed capacity import resources or generator-backed capacity import resources at each external interface. When this constraint is binding it will not determine the zonal capacity auction clearing price or zone group capacity auction clearing price. External interfaces not listed in the pre-auction report indicate that they border upon a jurisdiction where there is no associated agreement between the relevant balancing authorities.

system-backed capacity import resources and generator-backed capacity import resources will be subject to the zonal constraints in the external interface’s bordering electrical zone, as described in Section 2.6. As such, there will be no price separation for  either system-backed capacity import resources or generator-backed capacity import resources, and the resources located within Ontario’s electrical zone the interface borders. The IESO shall publish capacity import limits in the pre-auction report.

– End of Section -

# Pre-Auction Requirements

In order to conduct the capacity auction in a consistent and transparent manner, the IESO and the market participants must satisfy certain pre-auction requirements.

The IESO shall prepare a pre-auction report containing capacity auction related information, and publish it in advance of the auction, as explained in Section 3.1 below. There are also participant authorization, capacity qualification, and capacity auction deposit requirements for market participants who wish to participate in the capacity auction, as further explained in below.

## Pre-Auction Report

(Market Rules: Ch. 7, S. 18.5.2)

Prior to the capacity auction, the IESO shall publish a pre-auction report to include the following reference points, for both obligation periods:

- Target capacity

- Capacity auction reference price

- Minimum and maximum capacity auction clearing prices

- Minimum and maximum auction capacity to be secured

- Maximum auction capacity to be secured at the maximum capacity auction clearing price

- Zonal constraints for each electrical zone , as explained in Section 2.6 of this manual

- Capacity import constraints as explained in Section 2.7 of this manual

In addition to these reporting requirements, the IESO will also publish:

- the dates for capacity auction participants to submit a capacity qualification request

- the date that the IESO will notify capacity auction participants of the unforced capacity (UCAP) of their potential capacity auction resource ;

- the date by which capacity auction participants must post a capacity auction deposit;

- the dates that the IESO will conduct the capacity auction as well as the date by which the IESO will publish the public and confidential post-auction reports (Ch. 7, S. 18.5.4 of the market rules); and

- a link to a mapping tool to assist with the determination of which zone capacity auction resources are located, based on their physical address.

## Participant Authorization

(Market Rules: Chapter 2, section 2.1.1.1.11 and 2.1.1.1.12)

All prospective participants who wish to submit a capacity qualification request to participate in the capacity auction are required to authorize as capacity auction participants  through the IESO’s market registration process. The capacity auction participant shall authorize as a capacity market participant during the forward period if a capacity obligation is awarded, per Section 5.1.  Market authorization processes are further detailed in “Market Manual 1.5: Market Registration Procedures”.

In addition to authorization as a capacity auction participant, market participants may be authorized as one of the following classes described in Market Manual 1.5, as applicable:

- Generator

- Load

- Energy Trader

- Electricity Storage Participant

## Capacity Qualification

(Market Rules: Chapter 7, section 18.2 and 18.2A)

This section provides additional details with respect to the submission of a capacity qualification request by a capacity auction participant who wishes to participate in a given capacity auction, as well as the methodology the IESO uses to determine the unforced capacity (UCAP), in accordance with Chapter 7, sections 18.2 and 18.2A.

Authorized capacity auction participants who wish to participate in a given capacity auction must complete the capacity qualification process for each potential capacity auction resource. This process includes:

- the submission of a capacity qualification request including any additional data that’s required based on the capacity auction resource type as outlined in section 3.3.1 of this manual, and

- The IESO assessment of the potential capacity auction resources’ unforced capacity (UCAP) using the formulas outlined in this section .

Each potential capacity auction resource will represent a single resource according to the registration procedures described in “Market Manual 1.5: Market Registration Procedures”.

### Submission of a Capacity Qualification Request

(Market Rules: Chapter 7, section 18.2.1.2)

Prior to the deadline specified in the pre-auction report, authorized capacity auction participants wishing to participate in an upcoming capacity auction are required to submit to the IESO, via Online IESO, the following information as part of a capacity qualification request:

- The ICAP for each potential capacity auction resource.

  - Additional information required to determine the ICAP for capacity auction eligible storage resources is outlined in Table 1-1

  - For generator-backed capacity auction eligible import resources the ICAP must be provided for each generation unit associated with the generator-backed capacity auction eligible import resource

- The obligation period(s) for which they may wish to submit capacity auction offers. Participants may choose to submit capacity auction offers for one or both obligation periods,

- The type of capacity auction resource that will satisfy a capacity obligation during the commitment period and for HDR resources, the obligation type (e.g. physical or virtual) and contributor type (e.g. Residential or Commercial & Industrial). Refer to Section 5.3.2 for details on submitting demand response contributor data information.

Additional information is required for each capacity auction resource type, as specified in Table 1-1.

Table 1-1: Additional information required for submission of a capacity qualification request by capacity auction resource type.

| Capacity Auction Resource Type | Additional Information Required |
| --- | --- |
| Capacity generation resources | - The registered facility and associated resource that will satisfy the capacity obligation - A signed attestation declaring that the generator that will deliver auction capacity meets the requirements of a capacity auction eligible generation resource as set out in Chapter 11, Definitions of the market rules |
| Capacity storage resources | - The temperature-sensitive maximum power rating of the resource that can be sustained for 1 hour (Full Power Operating Mode) - The temperature-adjusted maximum amount of energy in MWh (Energy Rating), that the resource is capable of delivering when it is fully charged - The Full Power Operating Mode and Energy Rating will be used to determine the ICAP of a capacity storage resource using the following formula: ICAP = [min (Full Power Operating Mode, Energy Rating/4 hours)] - The registered facility and associated resource that will satisfy the capacity obligation - A signed attestation declaring that the generator that will deliver auction capacity meets the requirements of a capacity auction eligible storage resource as set out in Chapter 11, Definitions of the market rules |
| Capacity dispatchable load resources | - The registered facility and associated resource that will satisfy the capacity obligation - If there is no registered facility, the zonal location of the potential capacity dispatchable load resource. Participants may choose from the ten electrical zones to submit capacity auction offers |
| System-backed capacity import resources | - The external interface that will be used to deliver the auction capacity - A signed attestation acknowledging that all eligibility requirements have been met, as set out in Chapter 11, Definitions of the market rules and that has taken such actions as are necessary in order to ensure that: - Capacity imports related to a capacity obligation will be offered into Ontario’s energy market with firm 7F transmission service; and - The planning authority(ies) responsible for adequacy assessment(s) will remove any MW related to a capacity obligation associated with from its adequacy assessments |
| Generator-backed capacity import resources | - The external interface that will be used to deliver the auction capacity - The fuel type - The resource name - The unique numeric identifier for the generator-backed capacity auction eligible import resource - A signed attestation acknowledging that all eligibility requirements have been met, as set out in Chapter 11 of the market rules, that the capacity auction participant is the owner of the potential capacity auction resource, and proof of deliverability to the Ontario border in one of the following forms: - For generator-backed capacity auction eligible import resources located within the New York Independent System Operator (NYISO) control area, proof that the resource holds Capacity Resource Interconnection Service (CRIS) status; or - For generator-backed capacity auction eligible import resources located within the Hydro Quebec control area, confirmation of firm transmission service from the transmission operator |
| Hourly demand response (HDR) resources | - The registered facility and associated resource that will satisfy the capacity obligation - If there is no registered facility, the zonal location of the potential demand response resource and/or demand response contributors for which they are willing to submit offers, the obligation type (physical or virtual) and contributor type (Residential or Commercial & Industrial) |

### Capacity Qualification Assessment

(Market Rules: Chapter 7, section 18.2A.1 and 18.2A.2)

Based on the information provided by the participant as part of the capacity qualification request, the IESO will do the following to assess each capacity auction resource’s unforced capacity (UCAP):

- Verify that the market participant has completed the pre-auction authorization process to become a capacity auction participant as outlined in

- Ensure that the capacity market participant has not been disqualified from capacity auction participation as outlined in Ch.7 of the market rules, and

- Determine the maximum amount of UCAP that each capacity auction resource can offer into the capacity auction for one or both of the summer and winter obligation periods.

  - For generator-backed capacity auction eligible import resources, the IESO will assess the UCAP for each generation unit associated with the generator-backed capacity auction eligible import resource and provide a single UCAP that the capacity auction resource can offer into the auction.  The single UCAP will be equal to the sum of each individual resource’s UCAP.

The approach for determining the maximum UCAP for all capacity auction resource types can be generalized as follows:

UCAP = ICAP x Availability De-Rating Factor x (1 – PAF)

Where:

- Availability De-Rating Factor reflects a resource’s historic availability and is calculated for each capacity auction resource type as specified in table 1-2

- PAF is the Performance Adjustment Factor which reflects performance during a capacity test activation in a historic obligation period, and will be calculated as specified in table 1-2

Table 1-2: Availability De-Rating Factors and Performance Adjustment Factors by capacity auction resource type

| Capacity Auction Resource Type |  | Availability De-Rating Factor | Performance Adjustment Factor (PAF) |
| --- | --- | --- | --- |
| Capacity generation resources | Dispatchable Thermal | Availability De-Rating Factor = EFORd Where: EFORd is the equivalent forced outage rate on demand based on 5 years of historic data. For capacity generation resources with less than 5 years of historical data, a proxy value of 7% will be used based on the median EFORd of Ontario’s natural gas fleet | The PAF will be calculated according to the formula in Section 5.3 (Capacity Auction Capacity Tests), and will be based on the resource’s historical performance in a capacity auction capacity test in the most recent obligation period for which data is available, starting from the obligation periods associated with the 2022 capacity auction. |
|  | Dispatchable Hydro | Availability De-Rating Factor = Median of [(AQEI (MWh)+Scheduled Operating Reserve (MWh))/MAPC] in Top 200 hours of Ontario demand per season for the last 5 years Where: - AQEI is the Allocated Quantity of Energy Injected, in MWh - Scheduled OR is the Scheduled Operating Reserve in MWh - MAPC is the Maximum Active Power Capability, in MW, under any conditions without station service being supplied by the unit For capacity generation resources with less than 5 years of historical data, the median value for the zone will be used. Where there is no dispatchable hydro generation in a zone, an Ontario-wide median will be used. |  |
| Capacity storage resources |  | Availability De-Rating Factor = EFORd Where: EFORd is the equivalent forced outage rate on demand and is set at 5% |  |
| Capacity dispatchable load resources |  | Availability De-Rating Factor = Median(hourly bids quantity/maximum seasonal energy bid quantity) in top 200 hours of Ontario demand per season For new capacity dispatchable load resources, a fleet specific class median will be applied |  |
| Generator-backed capacity import resources | General | For generation units associated with a generator backed-capacity import resource that are located in a host control area with an accredited UCAP rating, the UCAP for the unit will be equal to the value determined by the host control area operator. |  |
|  | Dispatchable Thermal | For generation units associated with a generator backed-capacity import resource that are located in a control area without an accredited UCAP rating, the EFORd for the generation unit as calculated using IEEE standard 762 must be provided to be used as the availability de-rating factor in the UCAP calculation. |  |
|  | Dispatchable Hydro | For generation units associated with a generator backed-capacity import resource that are located in a control area without an accredited UCAP rating, production data for at least the last 1 year and up to the last 5 years including scheduled Operating Reserve must be submitted. The IESO will calculate the availability de-rating factor using the formula for a capacity generation resource (dispatchable hydro) located in Ontario and specified in this table. |  |
| Hourly demand response (HDR) resources |  | Not applicable |  |
| System-backed capacity import resources |  | Not applicable | Not applicable |

Upon completion of the assessment, the IESO will notify capacity auction participants, via Online IESO, of the maximum UCAP that can be offered into the capacity auction for each potential capacity auction resource.

Any capacity auction resource assessed to have a UCAP below 1MW will be unable to participate in the capacity auction. For generator-backed capacity auction eligible import resources, any generation unit associated with the generator-backed capacity auction eligible import resource that is assed to have a UCAP below 1 MW will be unable to participate as part of the generator-backed capacity auction eligible import resource in the capacity auction.

## Capacity Auction Deposit

(Market Rules: Chapter 7, section 18.2.1.3, 18.3, 18.4)

Following receipt of the unforced capacity (UCAP) from the IESO, all capacity auction participants wishing to submit capacity auction offers into the capacity auction are required to provide to the IESO a single capacity auction deposit. The submission of the capacity auction deposit is confirmed via Online IESO.

The purpose of this deposit is to establish the creditworthiness of the capacity market participant for auction activities. The pre-auction deposit is also intended to ensure that the capacity auction participant fulfills any post-auction and forward period obligations.

The IESO will calculate the capacity auction deposit amount that a capacity auction participant is required to submit for each obligation period, based on the  UCAP determined for each capacity auction resource and in each obligation period of the capacity auction.

The formula for calculating a capacity auction participant’s deposit amount in a capacity auction is as follows:

Capacity auction deposit = 	3% * (total UCAP * maximum auction clearing price per MW-day) * number of business days in obligation period

The IESO may impose a higher capacity auction deposit requirement depending on creditworthiness of the capacity auction participant in the IESO-administered market.

For capacity obligation transfers, the IESO will determine and notify the capacity transferee if additional capacity auction deposit funds are required, as determined in Section 8, to complete a transfer.

If additional capacity auction deposit funds are required, the formula for determining a capacity transferee’s deposit for a transfer is as follows:

Capacity auction deposit = 	3% * (transferred auction capacity * maximum auction clearing price per MW day) * number of business days in obligation period

However, the additional capacity auction deposit requirements from a transfer request may be satisfied by the capacity transferee’s existing capacity auction deposit, if it has not been refunded back to the capacity transferee.

The IESO will not pay interest on cash deposits.

Letters of credit must be submitted to the IESO in original hard copy form.

Capacity auction deposits by cash may be submitted by electronic funds transfer to an IESO-designated account.  Letters of credit must be submitted to the IESO in original hard copy form. The IESO will not pay interest on cash deposits.

The IESO will verify all submitted capacity auction deposits for participation in a capacity auction by:

- Reviewing the amount and type of deposit,

- Verifying that it meets the submission timing requirements, and

- Ensuring applicants are authorized as capacity auction participants.

The IESO will release the capacity auction deposit, at the capacity auction participant’s request, within five business days for:

- An unsuccessful capacity auction participant after the publication date of the post-auction report;

- A successful capacity auction participant when the capacity auction participant is authorized as a capacity market participant, sufficient capacity prudential support is posted, and a facility is registered to satisfy each of the capacity auction participant’s capacity obligations for each obligation period;

Upon completion of a successful capacity obligation transfer, the IESO will release all or a portion of a capacity transferor’s capacity auction deposit at the capacity transferor’s request, within five business days under the following conditions:

- The IESO will release the capacity auction deposit if the capacity transferor’s remaining capacity obligations are 0 MW; or has at least one resource registered and sufficient capacity prudential support is posted to meet the capacity auction participant’s capacity obligation in each obligation period in each of the cleared electrical zones; or

- The IESO will release a portion of the capacity auction deposit, if the above condition is not met, determined by the following formula:

Partial capacity auction deposit release = 	3% * (transferred auction capacity * maximum auction clearing price per MW day) * number of business days in obligation period

– End of Section –

# Auction Mechanics

The capacity auction mechanics involves a 3-stage process, as displayed in Figure 4-1 below:

Figure 4-1: Capacity Auction Mechanics Overview

## Stage 1: Offer Submission and Validation

Capacity auction participants are required to submit capacity auction offers via Online IESO, following the auction timelines detailed in Section 2.2 of this market manual. Each capacity auction participant may submit capacity auction offers associated with each potential capacity auction resource identified during the capacity qualification process for any quantity between 1 MW and the unforced capacity (UCAP) determined by the IESO during the pre-auction process, using offer laminations to reflect the price of providing the various levels of capacity.

Capacity auction offers must be submitted on an obligation period basis. A complete capacity auction offer includes a set of up to 20 monotonically increasing price-quantity pairs with the total offered quantity across all laminations equal to or less than the UCAP for the potential capacity auction resource. The capacity auction offer quantity must increase with every new lamination added to an offer set (Ch.7, S. 18.6.3.2 of the market rules).

A capacity auction participant may revise a capacity auction offer in Online IESO up until the capacity auction offer window closes, per the timelines detailed in Section 2.2 of this market manual.

A capacity auction offer will apply for the entire obligation period. The prices offered represent the minimum price at which the participant is willing to provide each incremental quantity of auction capacity.

A capacity auction offer must also specify, for each price-quantity pair, whether the entire auction capacity represented in the lamination must be cleared in full or whether it may be partially cleared (Ch.7, S. 18.6.3.4 of the market rules). A full flag indicates to the IESO that the capacity auction participant is only willing to clear the auction with the full amount of auction capacity offered in that lamination. A partial flag indicates to the IESO that the capacity auction participant is willing to clear the auction in 0.1 MW increments of the offer in that lamination.

The participant must be ready to provide auction capacity in the amount of their capacity obligation by the first day of the obligation period or be subject to non-performance charges as explained in Section 6 of this market manual.

## Stage 2: Auction Clearing

Once the capacity auction offer submission window closes, the IESO will review all capacity auction offers to determine the capacity auction clearing price for each zone, as per the timelines detailed in Section 2 of this market manual.

For each obligation period, the IESO shall determine  the capacity obligation for each capacity auction participant’s capacity auction resource (Ch. 7, S. 18.7.3 of the market rules), following the process stated below.

The IESO will consider all capacity auction offers and clear them against a downward-sloping demand curve, utilizing an optimization model to maximize the social welfare (i.e. the area under the demand curve less supply costs). This clearing process will respect all capacity auction zonal constraints and capacity import constraints. The clearing process will determine the capacity auction clearing price for each zone. When there is a capacity auction offer not selected, either partially or in full, due to the total maximum capacity auction zonal constraint, the capacity auction clearing price for that zone will be set at the lesser of:

- the price associated with the next economic quantity from a capacity auction offer in the same zone that would have cleared but for the total maximum capacity auction zonal constraint; or

- the Ontario-wide capacity auction clearing price.

The Ontario-wide capacity auction clearing price will be set equal to the price associated with demand curve for the quantity equal to the last-cleared price-quantity pair associated with a capacity auction offer. The total quantity cleared through a capacity auction may clear above the demand curve where doing so will maximize the overall objective function. An example of the auction clearing process, including zonal limitations, is shown in Figure 4-2.

Figure 4-2: Auction Selection Process with Zonal Limits

In the example illustrated in Figure 4-2, Zone 1 has a total maximum capacity auction zonal constraint of 150 MW. All offers are stacked by increasing price against the demand curve for the obligation period. As shown in the figure, after clearing the first offer of 80 MW from Zone 1, the auction engine can only partially clear the second offer (70 MW) at which point the total cleared quantity in Zone 1 is equal to the total maximum capacity auction zonal constraint. If the auction engine determines that the un-cleared quantity from the second offer in Zone 1 would have cleared but for the total maximum capacity auction zonal constraints, a zonal capacity auction clearing price will be determined, in the manner described above. The overall procurement will continue and the capacity auction offers will clear until the intersection with the demand curve at 340 MW, which will also set the capacity auction clearing price for Zone 2 & 3, and is also referred to as the Ontario-wide capacity auction clearing price.

If the IESO receives two or more capacity auction offers at the same price for the last available quantity, the capacity auction offer with the earlier time stamp shall be selected as the successful capacity auction offer (Ch.7, S. 18.7.5 of the market rules).

Once the capacity auction clearing price and quantity are set, the IESO shall determine for each obligation period, the capacity obligations for each capacity auction participant and its capacity auction resource(s) (Ch. 7, S. 18.7.4 of the market rules). Resources with a capacity obligation will be designated a capacity auction resource for the duration of the commitment period.

## Stage 3: Post-Auction Reporting Obligations

Once the auction has been cleared and auction capacity quantities and clearing prices are determined for all zones, the IESO will prepare public and private reports to communicate this information, as explained below.

The IESO shall publish public reports containing the following information for each obligation period (Ch.7, S. 18.8 of the market rules):

- The Ontario-wide capacity auction clearing price;

- The capacity auction clearing price for each zone;

- The amount of auction capacity acquired through the auction for each zone by obligation type (i.e. physical or virtual);

- The successful capacity auction participants that received a capacity obligation and their respective total capacity obligations in each zone; and

- The total amount ofunforced capacity (UCAP) of each capacity auction participant by obligation type and zone or external interface, as applicable.

The IESO will also issue confidential post-auction reports to each capacity auction participant with the capacity obligation(s) for each capacity auction resource, the capacity auction clearing price applicable to the capacity auction resource, and obligation period (Ch.7, S. 18.8.2 of the market rules).

If capacity obligations are modified as a result of a buy-out or capacity obligation transfer, the IESO will prepare public and confidential reports to communicate the information, as explained above.

– End of Section –

# Post-Auction Requirements

## Participant Authorization

There are post-auction authorization and registration requirements for capacity auction participants who have successfully cleared and secured one or more capacity obligations. Such participants are required to become authorized as capacity market participants (Ch. 7, S.18.2.3 of the market rules). This authorization enables participants to participate in the energy market to satisfy a capacity obligation.

In addition to authorization as a capacity market participant, market participants with generator-backed capacity import resources and system-backed capacity import resources must be authorized as an Energy Trader – Importer.

In the case of capacity market participants with system-backed capacity import resources or generator-backed capacity import resources, all participation contact roles must be assigned to the capacity market participant or to an affiliate of the capacity market participant. Details with respect to contact roles are set out in “Market Manual 1.3: Identity Management Operations Guide”.

Post-auction market participant authorization processes are further detailed in “Market Manual 1.5: Market Registration Procedures”.

### Prudential Support

All capacity auction participants with a capacity obligation are encouraged to post capacity prudential support for the obligation period, at least 60 days prior to the obligation period.

Further details on capacity prudential support requirements are outlined in “Market Manual 5.4: Prudential Support.”

## Registration Requirements

(Market Rules: Chapter 7, sections 19.2, 19.3, 19.6, 19.8, 19.9A, 19.10)

The following section describes the registration requirements for participation in the energy market. All registration requirements are initiated and completed in Online IESO.

In order to satisfy a capacity obligation in the energy market, a resource registered in the energy market must be assigned to each capacity obligation. The capacity auction resource may be assigned during the capacity auction participants’ submission of a capacity qualification request in the pre-auction period or during the forward period (upon completion of registration of the energy market resources) for demand response resources that did not exist at the time of capacity qualification. Any market participant seeking to register their facility and resource must follow the processes and timelines outlined in “Market Manual 1.5: Market Registration Procedures”.

This registration process must be completed at least 45 business days prior to the beginning of the obligation period for it to be effective as of the start of the obligation period. If the process is not completed by 45 business days before the start of the obligation period, the IESO cannot guarantee that the registration will be effective as of the start of the obligation period and this may have consequences related to non-performance charges. For clarity, under all circumstances, the registration process must be completed prior to the commencement of the obligation period or be subject to Ch. 7, S. 18.4.4 of the market rules.  Upon completion, the capacity market participant can assign the newly registered facility with their applicable capacity obligation.

Except in the case of a system-backed capacity import resource and generator-backed capacity import resource, the capacity market participant with a physical capacity obligation must be the registered owner, as described in “Market Manual 1.5: Market Registration Procedures”, of the registered facility associated with the capacity auction resource.  Capacity market participants participating with generator-backed capacity import resource must be the owner of the resource associated with the generator-backed capacity import resource as registered in the host system. Capacity market participants participating with virtual HDR resources may include physical or virtual non-dispatchable loads owned by a third party as demand response contributors.

Market participants that are seeking to change attributes of their resources (e.g., a resource may change its bid type in the IESO’s registration system), in the IESO’s registration system in order to satisfy a capacity obligation must complete the market registration process, including possible commissioning tests, 45 business days prior to the start of the obligation period for it to be effective as of the start of the obligation period. If the process is not completed by 45 business days before the start of the obligation period, the IESO cannot guarantee that the registration will be effective as of the start of the obligation period and this may have consequences related to non-performance charges. For clarity, under all circumstances, the registration process must be completed prior to the commencement of the obligation period or be subject to Ch. 7, S. 18.4.4 of the market rules.

Physical Demand Response Resource

To register a facility in accordance with “Market Manual 1.5: Market Registration Procedures”, a capacity market participant with a physical capacity obligation providing demand response capacity with a transmission connected load facility or with an embedded load facility that is revenue metered by the IESO must register their demand response resource as an HDR resource or as a dispatchable load (for example, a non-dispatchable load could be registered as an HDR resource). This facility registration includes the submission of demand response capacity.

A capacity market participant with a physical capacity obligation providing demand response capacity must register only one demand response resource for each capacity obligation.

Virtual Demand Response Resource

A capacity market participant with a virtual capacity obligation providing demand response capacity with a facility that is not revenue metered by the IESO must register their demand response resource as a virtual HDR resource and must register only one demand response resource for each capacity obligation.  Capacity market participants with a virtual HDR resource must indicate the contributor type associated with such virtual HDR resource (residential or commercial/ industrial/ institutional load type, as applicable).

Capacity market participants with a virtual capacity obligation participating with a virtual HDR resource may include multiple demand response contributors, provided such demand response contributors are of the same contribution type as the virtual HDR resource.  Demand response contributors for a virtual HDR resource may include multiple virtual (non-revenue metered) and/or physical (revenue metered) non-dispatchable load(s). More information on the contributor management process is detailed in Section 5.2.1.

A capacity market participant providing demand response capacity with both residential and commercial/industrial/institutional demand response contributors in the same zone must register two separate HDR resources in that zone (one for each contributor type).

### Contributor Management

As part of the contributor management registration process, the capacity market participant must submit individual demand response contributor information via Online IESO that will be associated with their registered virtual HDR resource(s). Each capacity market participant is responsible for maintaining its contributor registry throughout their obligation period.

The Online IESO interface allows capacity market participants to generate monthly contributor reports that provides a summary of their contributor participation information (resource IDs, meter point IDs, contributor type, and effective start/end dates), and corresponding capacity obligations secured under each of their respective virtual demand response resource(s).

The capacity market participant must submit their demand response contributor information through Online IESO within the specified submission window, but no later than the 14th business day prior to the start date of the effective month. Contributor registration requests will be processed and responded to by the IESO, including notice of approval or rejection, at least four business days before the start of the effective month. Rejections and/or failure to submit appropriate registration information by specified deadlines will defer the effective date of the changes to the next effective month. Refer to the latest Demand Response Contributor Management and Measurement Data Submission Timelines posted on the IESO public Website under Market Calendars.

Capacity market participants must also retain individual contributor meter data and all relevant supporting information for each respective contributor. The IESO may request such information in order to verify the accuracy of information disclosed by the capacity market participant at the time of an audit as detailed in Section 5.4.

There are two categories of demand response contributors that can be registered to meet a capacity obligation:

- Virtual HDR resources consisting of commercial, industrial, institutional and/or non-dispatchable loads (C&I) that can be classified as:

  - Virtual C&I HDR contributors; and

  - Physical C&I HDR contributors;

- Virtual HDR resources consisting of residential smart-metered loads that can be classified as:

  - Virtual residential HDR contributors

Virtual C&I HDR Contributors registration requirements:

For virtual C&I HDR contributors, the information must satisfy the following applicable requirements:

- Contributor name and physical address (street, city, province, postal code), where the physical address must be in the same electrical zone as the associated demand response resource

  - The capacity market participant may use the zonal map tool located on the IESO Zonal Map page (http://www.ieso.ca/zonal.map/index.html) to confirm the electrical zone for the associated demand response contributor;

- Applicable licensed Local Distribution Company (LDC) name, and LDC account number indicated on the demand response contributors’ LDC billing statement;

- Demand response contributor load class type (i.e., industrial, commercial, and/or institutional);

- Whether the demand response is to be provided via load interruption or behind-the-meter generation;

  - If demand response type is behind-the-meter generation, then the capacity market participant must specify the following generator name plate capacity information: model number, capacity in MW, fuel type and (if applicable) load following technology;

- Identification of whether the demand response contributor is participating in other demand response or conservation initiatives;

- Demand response capacity of contributor in MW;

- A declaration of acknowledgement by the capacity market participant that the LDC has been notified of the demand response contributors’ participation in a capacity auction;

- Data acquisition method used to collect demand response contributor meter data;

- Submission of LDC Billing statement for each LDC meter installation that is issued within three months of the demand response contributor effective date;

- Submission of single line diagram (SLD) is required when the demand response resource type is behind-the-meter generation. SLD submissions (at a minimum) must include the following details:

  - Facility/contributor name, physical address

  - Embedded connection point(s) (point of sale) to the local distribution company (LDC)

  - Location of distribution transformer

  - Location of breakers, disconnect switches, etc.

  - Location of the metering installation and meter point reference identification (as indicated on contributors’ Record of Installation)

  - If behind-the-meter generation, indicate generation location and nameplate information (MVA/kVA rating, output voltage)

Physical C&I HDR Contributors registration requirements:

For physical C&I HDR contributors, the information must satisfy the following applicable requirements:

- Non-dispatchable load Resource ID (subject to confirmation from non-dispatchable load owner); and

- Demand response capacity in MW.

As part of the contributor management process, any updates, revisions or amendments to demand response contributor information applicable to C&I HDR resources must be submitted using Online IESO for review and approval, including when:

- A new demand response contributor is added;

- An existing demand response contributor is removed; or

- An existing demand response contributor’s information is modified or amended.

- In instances when a new demand response contributor is added and/or an existing demand response contributor is removed, subject to IESO’s approval, the capacity market participant will be issued a new virtual meter point ID to reflect these changes. During a demand response activation event, the capacity market participant will be required to submit three months of measurement data under the issued virtual meter point ID, as detailed below.

- Virtual Residential HDR Contributors registration requirements:

For virtual residential HDR contributors, the information submitted to the IESO must satisfy the following applicable requirements.

Submitted on a monthly basis through Online IESO using an excel template (refer to Appendix A):

- Demand response contributor physical address (in the order of: street# & name, city, province, postal code), where the physical address must be in the same electrical zone as the associated demand response resource;

  - The capacity market participant may use the zonal map tool located on the IESO Zonal Map page (http://www.ieso.ca/zonal.map/index.html) to confirm the electrical zone for the associated contributor;

- Applicable licensed Local Distribution Company (LDC) name and LDC account number indicated on contributors’ LDC billing statement;

- Indicator flagging the control group demand response contributors, as defined in the section entitled “Randomized Control Trial Baseline Methodology” below, where there must be at least 350 control group demand response contributors which are chosen randomly (i.e. using a process of selection in which each contributor has an equal probability of being chosen) each month by the capacity market participant from the total population of demand response contributors under the residential HDR resource;

The following fields must be directly entered into the input fields in Online IESO:

- Demand response capacity in MW (note: the total capability from only the treatment group contributors and must be equal to or greater than 1 MW);

- Total number of demand response contributors in the treatment group as defined in the section entitled “Randomized Control Trial Baseline Methodology” below; and

- Total number of demand response contributors in the control group.

As part of the residential contributor management process, the capacity market participant shall use the excel template available in Online IESO (refer to Appendix A) to submit demand response contributor information on a monthly basis.

Rejections and/or failure to submit appropriate contributor management registration information each month by the specified deadlines will exclude the residential HDR resource to participate in the energy market (submit energy bids) for that month, and result in Availability Charges to be applied (as further described in “Market Manual 5, Part 5.5: Physical Markets Settlement Statements”).

Randomized Controlled Baseline Methodology

For HDR resources associated with either virtual or physical C&I contributors, performance is evaluated using a historical baseline (as described in Market Manual 5.5: Physical Markets Settlement Statements).

For HDR resources associated with virtual residential demand response contributors, a randomized controlled (RC) baseline methodology is used where two groups of contributors are established, as follows:

A “treatment” group, where demand response contributors are activated to provide demand response upon receipt of the demand response standby and activation notice; and

A randomized “control” group, where demand response contributors serve as a proxy for baseline consumption; therefore, are not activated to provide demand response. The “control” group demand response contributors are randomly selected using a process of selection in which each demand response contributor has an equal probability of being chosen each month.

The RC evaluates the consumption difference between the two groups of demand response contributors to determine the amount of demand response capacity delivered, as illustrated in Figure 5-1.

Figure 5-1: Randomized Control Trials (RC) performance evaluation

Refer to “Market Manual 5.5: Physical Markets Settlement Statements” for a further description of how residential HDR performance is evaluated and how settlements are calculated.

## Energy Market Participation

In order to satisfy their capacity obligation(s), capacity market participants will be required to submit dispatch data in the day-ahead commitment process as set out in “Market Manual 9.2: Submitting Operational and Market Data for the DACP”, and in the real-time market as set out in “Market Manual 4.2: Submission of Dispatch Data in the Real-Time Energy and Operating Reserve Markets.” Capacity market participants are required to follow dispatch instructions as set out in “Market Manual 4.3: Real-Time Scheduling of the Physical Markets.”

All capacity auction resources will be subject to test activations in the real-time market, as set out in Section 6.3.3. Capacity market participants with capacity obligation(s) allocated to HDR resources will be compensated for out of market activations, including capacity auction dispatch test activations, as detailed in “Market Manual 5.5: Physical Markets Settlement Statements.”

### Outage Management/ Non-Performance Events

Capacity market participants with a capacity auction resource, except for capacity market participants with system-backed capacity import resources, are required to submit outage requests as set out in “Market Manual 7.3: Outage Management.” Capacity market participants with HDR resources are required to inform the IESO of non-performance events as set out in Market Manual 7.3.

### Measurement Data Submissions

For Virtual C&I HDR resource (s):

Each virtual C&I HDR resource is associated with a virtual meter point ID that reflects demand response contributor changes to a capacity market participant’s virtual portfolio. Capacity market participants are required to submit three months of aggregated measurement data (on a five-minute interval basis) through Online IESO only for months in which they are activated for their demand response capacity obligations. The Online IESO data submission must include measurement data for the activation month and two previous months of historical data in a single three-month data file per virtual meter point ID.

Processing of Measurement Data

Virtual C&I HDR resource will have either a uni-directional meter (kWh delivered) or a bi-directional meter (kWh delivered and kWh received). Capacity market participants must adhere to the following methodology when aggregating demand response contributor meter data and submitting a consolidated three-month measurement data file:

- Virtual demand response contributors with a uni-directional meter type, the uni-directional interval meter readings will be recorded in the summation of Channel 1 (kWh delivered) energy quantities. Channel 2 (received) energy is recorded as zero for that demand response contributor

- Virtual demand response contributors with a bi-directional meter type, the demand response contributor’s bi-directional interval meter readings must be netted (kWh delivered – kWh received) and recorded as follows:

  - if the resultant net kWh quantity is less than or equal to zero, then the total net kWh value will be zero and is recorded in the summation of Channel 1 (delivered) energy quantity for that interval. Channel 2 (received) energy is recorded as zero for that interval, or

  - if the resultant net quantity is greater than zero, then the total net value will be equal to the net amount and will be included in the summation of Channel 1 (delivered) energy quantity for that interval. Channel 2 (received) energy is recorded as zero for that interval

The measurement data submission is the summation of all demand response contributors by channel per interval.

File Format Requirements for Measurement Data Submissions

Measurement data submitted by capacity market participants through Online IESO, must adhere to the following requirements:

- Must not include any measurement error corrections,

- Must not include any loss adjustments,

- Must be provided in the following format:

  - A CSV (comma separated values) file format compatible with the IESO’s Meter Data Acquisition System, containing two channels of 5-minute engineering unit values (without any gaps or overlaps).

  - The CSV data file shall adhere to the following format (separated by commas) corresponding to each column name, as illustrated in Figure 6-2 below,

    - Row 1 (Main header): “DATE,TIME,CH1,CH2”

    - Row 2 (Data intervals): “YYYY/MM/DD, HH:MM, ###.###,###.###”, where:

      - Date: “YYYY/MM/DD”, as in year/month/day

      - Time: “HH:MM”, hour: minutes in Eastern Standard Time (EST),

      - Channel 1: Summation of all virtual contributors’ energy withdrawn from the grid, in Numeric “###.###,” in kWh up to three decimal places,

      - Channel 2: Summation of all virtual contributors’ energy injected into the grid, in Numeric “###.###,” in kWh up to three decimal places, and

  - The CSV data file must contain 288 rows of data per day, having a beginning time of 00:05 and an end time of 24:00.

Figure 5-2: Sample CSV File Format for Measurement Data Submission for C&I HDR

For Virtual Residential HDR resource (s):

Capacity market participants are required to submit aggregated hourly (60-minute interval) measurement data only for days in which they received demand response activations during the commitment month. Measurement data (single data file per virtual meter point ID for all activation days) must be submitted for each of the two groups of demand response contributors (treatment and control group) through Online IESO in accordance with the latest Contributor Management Timelines posted on the IESO public Website under Market Calendars.

Subject to IESO’s approval, the capacity market participant will be assigned two unique Meter point IDs (MPID), one for the treatment group and one for the control group. The MPID format for each group is as follows:

- DRAT########## to represent the treatment group demand response contributors, and

- DRAC########## to represent the control group demand response contributors.

File Format Requirements for Measurement Data Submissions

Measurement data submitted by capacity market participants through Online IESO must adhere to the following requirements:

- Must not include any measurement error corrections,

- Must not include any loss adjustments,

- Must be provided in the following format:

  - A CSV (comma separated values) file format containing two channels of 60 minute engineering unit values (without any gaps or overlaps),

  - The CSV data file shall adhere to the following format (separated by commas) corresponding to each column name, as illustrated in Figure 5-3 below,

    - Row 1 (Main header): “DATE,TIME,CH1,CH2”

    - Row 2 (Data intervals): “YYYY/MM/DD, HH:MM, ###.###,###.###”, where:

      - Date: “YYYY/MM/DD”, as in year/month/day

      - Time: “HH:MM”, hour:minutes in Eastern Standard Time (EST),

      - Channel 1: Summation of all virtual contributors’ withdrawn energy in kWh up to three decimal places, in numeric value “###.###”,

      - Channel 2: Shall remain zero (with respect to the exclusion of ‘net-metered’ customers under residential HDR),

  - The CSV data file must contain 24 rows of data per day, having a beginning time of 01:00 and an end time of 24:00.

Figure 5-3: Sample CSV File Format for Measurement Data Submission for Residential HDR

Timelines for Data Submission and Processing

Upon activation, capacity market participants must submit their measurement data no later than the 6th business day before the end of the subsequent month. Refer to the latest Demand Response Contributor Management and Measurement Data Submission Timelines posted on the IESO public Website under Market Calendars for details.

The IESO will process all measurement data submissions and respond to the capacity market participant with notice of any errors by the 4th business day prior to the start of the effective month. The capacity market participant will then have (at a minimum of) two business days from the date the IESO provides such notice to correct and resubmit a revised measurement data file through Online IESO. Measurement data submissions not submitted by the specified deadlines will incur non-performance charges in accordance with Ch. 9, S. 4.7J of the market rules.

Capacity market participants must retain individual demand response contributors measurement data and all supporting information provided at the time of registration, for audit purposes for a period of seven (7) years. The IESO may request such information in order to verify the accuracy of information disclosed by the capacity market participant.

#### Validation, Estimation and Editing (VEE) Process for Virtual C&I HDR Contributors

For virtual C&I HDR contributors, if the capacity market participant has identified, within the measurement data submission deadline, that a portion of the measurement data is missing for particular demand response contributor (s), the capacity market participant shall:

- Collect data for all demand response contributors for the period of three months excluding the missing period

- Utilize the following Validation, Estimation and Editing (VEE) criteria for virtual C&I HDR contributors to account for the missing period:

  - If the data is missing for any period outside the hours of a demand response activation event; measurement data for the missing period will be estimated to zero.

  - If the data is missing for any period within the demand response activation event; the capacity market participant shall take the highest 5 min interval energy value (kWh) from the entire three-month data set and estimate the missing period with that value.

Capacity market participants must submit a “Measurement Data Control Sheet” with each measurement data submission identifying demand response contributors with VEE data (if applicable). A template of the “Measurement Data Control Sheet” can be found in Appendix B.

At the time of an audit, the IESO shall take into account all supporting information provided by the capacity market participant including measurement data submitted during the commitment period, the actual measurement data submitted at the time of the audit along with the measurement data control sheet (if applicable).

### Testing of Capacity Auction Resources

#### Capacity Auction Capacity Tests

(Market Rules: Chapter 7, sections 19.4.11.A, 19.5.7A, 19.7.7A, 19.11.7.A)

The IESO will direct applicable capacity auction resources to perform one capacity auction capacity test per obligation period.

HDR Resources, Capacity Dispatchable Load Resources, Capacity Generation Resources, and Capacity Storage Resources

To execute a capacity auction capacity test, the IESO will determine a 5-business day window (“testing window”) for each obligation period within which applicable capacity auction resources will be required to ensure they are scheduled in the energy market for a resource-specific amount of time within the applicable availability window. The IESO will issue an advisory notice indicating when the testing window will be scheduled, a minimum of 5 business days in advance of the testing window.

Capacity auction capacity tests for these capacity auction resources are conducted as follows:

- Capacity auction resources must submit energy market bid/offer prices to ensure they receive a generation (for generators and storage) or load curtailment (for dispatchable load and HDR) schedule/activation in the energy market to the greater of their cleared ICAP or their minimum loading point (for capacity generation resources) during the testing week within the availability window for the following durations:

  - Capacity generation resources (quick start and non-quick start generators), capacity storage resources, and hourly demand response resources (HDRs) – 4 consecutive hours

  - Capacity dispatchable loads – 3 dispatch intervals

Note – the demand response bid price thresholds for HDRs and capacity dispatchable loads will be removed for all hours of availability during the testing window.

- Capacity market participants must notify the IESO, no later than 5 business days after the end of testing window, of the specific day, hours and dispatch intervals, if applicable, for which they wish their performance to be assessed.

Failure of a capacity market participant to notify the IESO by the deadline may result in non-performance charges as specified in “Market Manual 5.5: Physical Markets Settlement Statements”.

The IESO will assess the performance results of the capacity auction capacity test using the following parameters:

- The capacity auction capacity test will be deemed a pass if the capacity auction resource delivered, on average for its specified test duration, its cleared ICAP within the specified threshold of 10% for HDR resources and 5% for all other resource types.

- If the test is deemed a pass, the Performance Adjustment Factor (PAF) applied to the capacity auction resource in the next future capacity auction in which it participates will be 0.

Failure of the capacity auction capacity test will result in the applicable non-performance charges as specified in “Market Manual 5.5: Physical Market Settlements”. The PAF to be applied to the next future capacity auction in which the capacity auction resource participates will be calculated according to the formula below:

PAFsummer or winter = 1 – (delivered MW/cleared ICAP)

Where:

Delivered MW is equal to the average amount of auction capacity delivered by the capacity auction resource over its specified test duration.

If data is missing to calculate the “delivered MW” (i.e. the capacity market participant did not notify the IESO of the completed capacity auction capacity test by the deadline), then the “delivered MW” will be set at 75% of the capacity auction resource’s cleared ICAP.

Generator-Backed Capacity Import Resources

To complete a capacity auction capacity test for a generator-backed capacity import resource, the capacity market participant must:

- Submit data to the IESO confirming the capability of all generation units associated with the generator-backed capacity import resource to inject, in aggregate, the generator-backed capacity import resource’s cleared ICAP into the grid of the host control area in which it is located for 4 consecutive hours within the availability window, on a date that falls within the first two months of the applicable obligation period, and

- Successfully schedule an import transaction at the cleared ICAP from the host control area into Ontario at the relevant intertie for at least 1 hour that coincides with the timing of the scheduled 4-hour activation in the host control area being used to confirm capability of the generator-backed capacity import resource.

Data Submission Requirements

The data submission must consist of revenue-grade quality meter data that meets the market participation requirements of the host control area operator in which it is located. The data shall be provided on a 5-minute interval basis and shall be verified as complete and accurate by the host control area operator.  The data submission must be sent to capacity.auction@ieso.ca.

Timelines for Data Submission and Processing

The data set must be provided to the IESO no later than the sixth (6th) business day before the end of the month following the month in which the monthly data set relates.  The IESO will review all data submissions and respond to the capacity market participant with notice of any errors or clarifications by the fourth (4th) business day before the end of the month following the month in which the data set relates.  The capacity market participant will then have (a minimum of) 2 business days from the date the IESO provides such notice to respond, or correct and resubmit the data file to capacity.auction@ieso.ca.

Failure of a capacity market participant to provide the data set in a timely, complete or accurate manner may result in non-performance charges as specified in “Market Manual 5.5: Physical Markets Settlement Statements”.

Performance Assessment

The IESO will assess the results of the capacity auction capacity test using the following parameters:

- If the generator-backed capacity import resource demonstrates, through their data submission, an injection of electricity into the host control area’s grid equal to their cleared ICAP within a threshold of 5% for 4 consecutive hours within the required date and time, and successfully scheduled the corresponding import for its cleared ICAP into Ontario for at least 1 hour during the same time period, the test will be deemed a pass.

- If the test is deemed a pass, the Performance Adjustment Factor (PAF) applied to the capacity auction resource in the future auction will be 0.

- Failure of the capacity auction capacity test will result in the applicable non-performance charges as specified in “Market Manual 5.5: Physical Market Settlements”. The PAF (specific to winter/summer) will be applied to the future capacity auction qualification, and will be calculated using the following formula:

PAFsummer or winter = 1 – (delivered MW/cleared ICAP)

Where:

Delivered MW is equal to the average amount of auction capacity delivered over the entire duration of the test, which in the case of generator-backed capacity import resources would be the average MW output of the generation units associated with the generator-backed capacity import resource in the host control area over the course of the test hours as specified by the capacity market participant.

If supporting data is missing to calculate the “delivered MW” (i.e. data was not submitted), then the “delivered MW” will be set at 75% of the generator backed capacity auction resource’s cleared ICAP.

System-Backed Capacity Import Resources

The IESO may direct system-backed capacity import resources to perform up to two capacity auction capacity tests per obligation period for each capacity auction resource to verify that the auction capacities are deliverable. Tests will be scheduled to occur during the availability window of the dispatch day.

The tests are conducted as follows:

- Up to 2 hours in advance of any test, applicable system-backed capacity import resources will receive a constraint and a schedule should appear in PD-1. Tests may be scheduled for a duration of up to 4 hours.

- If a system-backed capacity import resource being tested is successfully scheduled in pre-dispatch and not curtailed and subject to a capacity charge as per “Market Manual 4.3: Real-Time Scheduling of the Physical Markets”, the test will be deemed a success.

- Failure of the test will result in the applicable charges as specified in “Market Manual 5.5: Physical Markets Settlement Statements.”

If a system-backed capacity import resource is unable to comply with the test activation of the auction capacity on the dispatch day, it is the responsibility of the capacity market participant to notify the IESO, and update the energy offers in accordance with “Market Manual 4.2: Submission of Dispatch Data in the Real-Time Energy and Operating Reserve Markets”. A subsequent test activation will be rescheduled by the IESO following the completion of the outage.

The IESO may determine a test activation for a system-backed capacity import resource is not required if all the following criteria are met. The:

- system-backed capacity import resource receives and follows sufficient dispatch instructions in the energy market,

- dispatches are within the availability window, and

- system-backed capacity import resource demonstrates that its capacity obligation has been met.

Failure of a system-backed capacity import resource to perform a successful test activation may result in one or more of the following:

- Non-performance charges as specified in “Market Manual 5.5: Physical Markets Settlement Statements,”

- A subsequent test activation to be scheduled by the IESO, or

- A compliance investigation to be performed by the IESO.

#### Capacity Auction Dispatch Tests

The IESO reserves the right to perform up to two capacity auction dispatch tests per obligation period to validate a capacity auction resource’s ability to deliver their scheduled capacity with minimal notice. Capacity auction dispatch tests for all applicable capacity auction resource types will be scheduled for a duration of up to 4 hours within the availability window of the dispatch day.

The IESO may determine a capacity auction dispatch test for a capacity auction resource is not required if all the following criteria are met:

- the capacity auction resource has demonstrated its ability to follow dispatch instructions within compliance dead-bands in the energy market, and

- dispatches are within the availability window.

The IESO may schedule a capacity auction dispatch test for a capacity auction resource regardless of whether the above conditions are met, if there is evidence that the capacity auction resource is not able to follow dispatch instructions at any time during the obligation period.

A capacity auction resource that fails their capacity auction dispatch test may be subject to one or more of the following:

- Non-performance charges as specified in “Market Manual 5.5: Physical Markets Settlement Statements.”

- A subsequent dispatch test to be scheduled by the IESO, or

- Referral to the Market Assessment and Compliance Division.

Capacity Generation Resources, Capacity Storage Resources, and Capacity Dispatchable Load Resources

Capacity auction dispatch tests for these capacity auction resources will be conducted as follows:

- The IESO will apply a constraint to the resource’s offer MW amount in advance of the capacity auction dispatch test up to:

  - 1 hour for capacity generation resources that are quick start generators and capacity storage resources.

  - 1 business day for capacity generation resources that are non-quick start generators in the day-ahead timeframe and prior to the day-ahead commitment process.

- Capacity dispatchable loads will have a constraint applied to curtail up to their bid MW amount up to 1 hour in advance of the capacity auction dispatch test.

If the capacity auction resource demonstrates it was able to follow its dispatch instructions within the applicable compliance dead band as per the Market Rule Interpretation Bulletin (IMO_MKRI_0001), the test will be deemed a pass.

If the capacity auction resource is unable to comply with the capacity auction dispatch test in accordance with their offered/bid MW amount on the dispatch day, it is the responsibility of the capacity market participant to notify the IESO, according to the outage reporting requirements for the applicable capacity auction resource type as specified in “Market Manual 7.3: Outage Management” and update the energy offers/bids in accordance with “Market Manual 4.2: Submission of Dispatch Data in the Real-Time Energy and Operating Reserve Markets”. A subsequent capacity auction dispatch test may be rescheduled by the IESO following the completion of the outage.

Hourly Demand Response Resources

Capacity auction dispatch tests for HDR resources are conducted as follows:

- Capacity market participants with HDR resources will receive a standby notice on the pre-dispatch day and an activation notice approximately 2 hours and 30 minutes in advance (but no later than 2 hours in advance) of the first dispatch hour of the capacity auction dispatch test. Resources will receive a schedule in pre-dispatch and real-time, regardless of the demand response energy bid price submitted.

If the HDR resource being tested demonstrates a curtailment of electricity withdrawal from the IESO-controlled grid equal to their demand response energy bid within a 15% dead-band for every dispatch interval (or every dispatch hour for residential HDRs) for the duration of the test (up to 4 hours), the test will be deemed a pass.

If the HDR resource being tested is unable to comply with the capacity auction dispatch test on the dispatch day, it is the responsibility of the capacity market participant to manage its non-performance as described in “Market Manual 7.3: Outage Management”. If the non-performance event indicates that the entirety of the HDR resource’s demand response energy bid is unavailable, a subsequent capacity auction dispatch test may be rescheduled by the IESO following the completion of the non-performance event.

An HDR resource’s capacity auction dispatch test is considered valid, unless:

- The capacity market participant provides notice of a non-performance event that would reduce its demand response energy bids to 0 MW,

- The IESO did not send either a standby or activation notifications in advance of the test as per the timelines specified above, or

- The IESO cancels the test prior to the start of the first dispatch hour of the test activation. The IESO will appropriately inform capacity market participants with HDR resources about the test cancellation.

Performance Parameters:

Performance of an HDR resource means the demand response energy bid for the HDR resource is delivered for each hour of the activation period within a 15% dead-band (e.g. at least 85% of the demand response energy bid must be delivered).

Performance will be assessed using the following parameters:

- The load reduction, up to a maximum of 115% of an HDR resource’s energy bid quantity, will be considered per 5-minute dispatch interval

The load reduction across each 5-minute dispatch interval will be summed for each activation hour (all 12 intervals) to determine the hourly load reduction

## Measurement Data Audit for Virtual HDR Resources

The IESO conducts audits to assess and verify the completeness and accuracy of submitted demand response measurement data, and supporting information and documents including but not limited to the Local Distribution Company billing statements, and Single Line Diagrams. The audit procedures and processes described herein are specific to the Virtual C&I HDR resources.

### Capacity Market Participant’s Responsibilities

This section covers the capacity market participants’ responsibilities associated with performing measurement data audits.

The capacity market participant is responsible for:

Providing the IESO auditor with access to the information required;

Submitting information and evidence requested; and

Payment of non-performance charges, as outlined in Section 6, if the audit requirements are not met

### Virtual C&I HDR Resource Audit

Virtual C&I HDR resource audit will be conducted by evaluating each demand response contributor that is mapped to the selected Virtual C&I HDR resource. The IESO will establish audit results by conducting a review of the supporting information provided at the time of registration and documentations provided during the audit including: Local Distribution Company (LDC) billing statements and individual demand response contributor measurement data for the respective virtual C&I HDR resource. All processes related to the virtual C&I HDR resource audit will be managed through the Online IESO.

### Audit Scheduling and Submission of Supporting Documents

The Virtual C&I HDR Resource audit can be categorized as follows:

- Full Audit

  - Capacity market participants are required to submit all required documents for all demand response contributors

- Partial Audit

  - A spot check to evaluate and compare meter data interval(s) for one or more demand response contributors against their respective LDC interval meter data; or

  - A manual selection of a set of demand response contributors from a portfolio. In case of a manual selection, capacity market participant is required to submit all required documents for the selected demand response contributors.

The default deadline is set to one calendar month from the date of issuance for the submission of all required supporting documentation:

Local Distribution Company Billing Statement

The capacity market participants are required to provide to the IESO a copy of Local Distribution Company (LDC) billing statement for all the demand response contributors registered under the capacity market participant’s portfolio. This information will be used by the IESO auditor to verify:

- The LDC account number with the information found in the meter registry.

- The total energy presented on the LDC statement against the meter data file submitted for the individual demand response contributor.

Measurement Data

The capacity market participants are required to provide the IESO with individual demand response contributor meter data as explained in section 5.3.2. The IESO auditor will assess the following criteria at the time of audit:

- The participant is available to curtail its load on business days and hours during an obligation period as defined in this manual.

- The participant has submitted measurement data for the audit month and an additional two months of baseline.

- Actual measurement data meets the criteria defined in Section 5.4.4 of the document.

### Procedure to Conduct a Virtual C&I HDR Audit

The Virtual C&I HDR audit consists of two steps:

- Step 1 of the audit reconciles actual demand response contributor measurement data to the demand response contributors LDC billing statement

- Step 2 of the audit reconciles the sum of the demand response contributor’s actual measurement data to the submitted measurement data (this is the measurement data provided by the capacity market participant during activation months in accordance with the Demand Response Measurement Data Submission Timelines).

Mechanism for Step 1 of the Audit Process

To determine the error in Step 1 of the audit process; the sum total of the actual measurement data file for a single contributor is compared against the total monthly consumption indicated in the LDC billing statement for that contributor. The difference between the two values shall be within 1% of the consumption indicated in the LDC Statement.

Step 1 of the audit process consists of two individual reconciliation checks

- Comparing the total kWh (energy) for a given month – Area under the curve

- Comparing the highest kW (Power) value – Peak Demand

These reconciliation checks verify the contributor’s data against the total monthly consumption and the peak demand indicated on the LDC statement. However, these reconciliation checks do not provide adequate assurance that the data will reconcile on an interval by interval basis. As such, the IESO at its discretion, may request the capacity market participant to provide 5 min LDC interval data with a declaration stating that the data has been collected from the LDC. This data will then be compared against the data provided by the capacity market participant as part of the audit request. An audit can be deemed as “Complete with Observations” if the intervals from the submitted measurement data are outside the +/-1% threshold when compared to intervals from the LDC verified 5 min interval meter data.

Mechanism for Step 2 of the Audit Process

IESO uses Absolute Error methodology to determine the error in Step 2 of the audit process. The methodology is described below:

- At the time of the audit of a resource, the aggregator is required to submit actual meter data for each contributor that makes up that resource.

- The actual data is then compared to the submitted measurement data on a 5 min interval basis.

- An absolute difference between the actual measurement data and submitted measurement data is taken.

- Sum of the absolute difference is compared against the sum of the submitted measurement data.

- This sum of the absolute difference should be within 1% of the summed submitted measurement data.

### Audit Review and Remedial Actions

The IESO will review supporting documents submitted by the capacity market participant for completeness and accuracy. If the review produces any findings, the capacity market participant shall be required to submit remedial evidence within the prescribed period as per the audit outcome. If findings are not resolved after one resubmission, the IESO shall close the audit with observations and determine a course of action in order to enforce compliance.

### Closure of Audit

Once the review of the submitted evidence is complete, the IESO will disclose the audit results to the capacity market participant and close the audit as follows:

- Virtual C&I HDR Resource audit is considered ‘Complete’ when

  - Contributors actual measurement data reconciles with associated LDC billing statement (tolerance of +/- 1%) and

  - Sum of actual measurement data reconciles with submitted measurement data (tolerance of +/- 1%)

- A Virtual C&I HDR Resource audit is ‘Closed with Observations’ when it is concluded that actual measurement data and supporting documentation differs from submitted measurement data and supporting documentation (Ch. 9 s.4.7J.4) i.e. that the audit reveals that data was outside the prescribed threshold in either Step 1 or Step 2 of the audit process.

– End of Section –

# Settlements

Capacity market participants with capacity obligations will be settled, for both payments and non-performance charges, using the physical markets settlement process as detailed in “Market Manual 5.5: Physical Markets Settlement Statements.” Details on how the costs will be recovered are also provided in Market Manual 5.5.

Capacity market participants will be paid availability payments, and may be eligible for test activation/emergency operating state activation payments and availability charge true-up payments, as detailed in Market Manual 5.5. Applicable non-performance charges will apply when energy market participation requirements outlined in this manual are not met.

In general, non-performance charges occur for the following situations:

- Availability requirements are not met (i.e. availability charge);

- Measurement data submission was not accurate, timely or complete (i.e. administration charge);

- Dispatch instructions were not followed (i.e. dispatch charge); and

- Failing a capacity auction capacity test (i.e. capacity charge).

Non-performance charges will be calculated and settled as detailed in Market Manual 5.5.

## Non-Performance Factors

The non-performance factors referenced in Market Manual 5.5 will use the factors listed in the table below for settling each capacity obligation for the month that is being settled.

Table 6-1: Non-Performance Factors

| Month | Factor |
| --- | --- |
| January | 2.0 |
| February | 2.0 |
| March | 1.5 |
| April | 1.0 |
| May | 1.0 |
| June | 1.5 |
| July | 2.0 |
| August | 2.0 |
| September | 2.0 |
| October | 1.0 |
| November | 1.0 |
| December | 1.5 |

– End of Section –

# Buy-out Process

Successful capacity auction participants and capacity market participants have the option to buy-out of their capacity obligations at any time. A full or partial buy-out may be requested anytime during the forward period or the obligation period. The buy-out will be valid from the effective date of the buy-out request until the end of the associated obligation period. The effective date will be no sooner than the two business days following the date that the IESO receives the request to buy-out. Upon IESO’s acceptance of a buy-out request, a buy-out charge will apply and is settled using the physical markets settlement process for the next available month-end preliminary settlement statement. Participants may refer to “Market Manual 5.5: Physical Markets Settlement Statements” for details on how the buy-out charge is calculated prior to initiating the buy-out process.

In order to initiate a buy-out, a written request must be submitted to the IESO by the registered capacity auction contact via email to: customer.relations@ieso.ca.

The email must contain the following information for each capacity obligation the participant requests to buy-out from:

- Capacity obligation ID;

- Effective date of the buy-out request;

- Buy-out obligation period: Specify the obligation period the buy-out is being requested for;

- Buy-out zone;

- Capacity auction resource; and

- Buy-out capacity: Specify the capacity of the buy-out request in MW, to one decimal place. In the case of a partial buy-out request, the remaining capacity obligation must be greater than or equal to 1 MW. In the case of a full buy-out request, the remaining capacity obligation must be 0 MW.

The IESO will process the buy-out request within seven business days. At the end of this review period, the IESO will either:

- Approve the buy-out request: The IESO will notify the participant of the applicable buy-out charge.

  - If the participant has requested for a partial buy-out, the IESO will notify it of the revised capacity obligation.

  - If the participant has requested a full buy-out of all its capacity obligations, the IESO will refund its pre-auction deposit amount, at the participant’s request, within ten business days after the IESO has received payment for the buy-out charge.

  - If the capacity prudential support obligation is revised downward due to a buy-out, the IESO will refund the difference, at the participant’s request, after the IESO has received the payment for the buy-out charge. The revised capacity prudential support obligation will be based on the revised capacity obligation.

OR

- Reject the buy-out request: The IESO will provide a reason for rejection.

The buy-out takes effect upon the IESO’s communication to the participant confirming that the buy-out request has been approved. For greater clarity, a participant should not begin adjusting their energy market bids in advance of receiving the IESO’s approval otherwise they may be subject to non-performance charges as specified in Market Manual 5.5: Physical Market Settlements.

– End of Section –

# Capacity Obligation Transfer

Capacity auction participants and capacity market participants may transfer their capacity obligations fully or partially. Once approved by the IESO, the capacity obligation transfer will be effective as of the first day of the associated obligation period and will be valid for the entirety of that obligation period.

A capacity transferor may request a full or partial capacity obligation transfer during the forward period, provided such request is made no later than 14 business days prior to the start of the obligation period.

In order to initiate a capacity obligation transfer, a request must be submitted to the IESO using Online IESO by the capacity transferor. A separate request must be submitted for each capacity obligation and contain the following information:

- Capacity obligation ID and associated capacity auction resource belonging to the capacity transferor;

- The name of the capacity transferee;

- The capacity (in MW) of the transfer request. For both transferee and transferor, the respective resulting capacity obligations cannot be between 0 and 1 MW (but, for greater certainty, can be 0 MW and can be equal to or greater than 1 MW); and

- The obligation period for which the transfer is being requested;

The request will not be considered by the IESO until the IESO receives, via Online IESO, confirmation from the capacity transferee that it accepts the new/additional capacity obligation (only when the capacity transferor and the capacity transferee are not the same capacity auction participant) and the name of the capacity auction resource that will accept the capacity obligation. If a capacity auction participant or a capacity market participant intends to submit multiple partial transfer requests in relation to a single capacity obligation, only one transfer request may be submitted at a time.  The first request submitted must be approved by the IESO before the second request can be submitted.

- The IESO will assess each capacity obligation transfer request in the order received by the IESO and determine whether the capacity obligation transfer request meets the criteria stipulated in Chapter 7, Section 18.9 of the market rules. These criteria include satisfying any revised capacity prudential support obligation or capacity auction deposit, as applicable:

- A revised capacity prudential support obligation is required if the capacity obligation to be transferred will be satisfied by the capacity transferee’s existing resource that is registered to meet a capacity obligation for the same obligation period and for which sufficient capacity prudential support, prior to the transfer, has been posted, otherwise;

- A revised capacity auction deposit is required.

- In either case, the capacity transferee must satisfy any revised capacity prudential support obligation (as specified in Section 1.3.8 of Market Manual 5.4), or the revised capacity auction deposit (as specified in Section 3.4), within five business days of receiving notification from the IESO of such requirement, or such longer period as agreed upon between the IESO and the capacity transferee. The IESO will notify the capacity transferee of any additional capacity auction deposit or capacity prudential support obligation, as required.

- After all criteria are assessed, the IESO will approve or reject the capacity obligation transfer. If rejected, the IESO will provide a reason for rejection to both the capacity transferor and the capacity transferee.

- If approved, the IESO will notify the capacity transferor and the capacity transferee. If the capacity transferor has requested a partial transfer, the IESO will notify the capacity transferor of the revised capacity obligation.

- A capacity transferee who acquires a capacity obligation as a result of a transfer from a different zone will be settled based upon the capacity auction clearing price received when that first originally cleared the capacity auction (i.e. the original zone). The revised capacity auction clearing price will be included in the participant’s confidential post-auction  report. For example, a capacity auction participant receives a capacity obligation of 25MW in a capacity auction at a capacity auction clearing price of $100/MW-day. If the capacity auction participant accepts a capacity obligation transfer for an additional auction capacity of 50MW for the same capacity auction resource via a transfer from another zone where the capacity auction clearing price is $40/MW-day, the revised capacity obligation for the capacity auction resource will be 75MW. Its revised (blended) capacity auction clearing price will be $60/MW-day, calculated from [(25 x $100) + (50 x $40)] ÷ 75 = $60/MW-day.

- Upon completion of a successful transfer, the capacity transferor may request to reduce its capacity auction deposit, if applicable, as specified in Section 3.4.

– End of Section –

Appendix A: Template for DR Residential Contributor Management Registration

This excel template is available in Online IESO under residential contributor management:

| Contributor Address | LDC Name | LDC Account# | Control Group Flag (Y/N) |
| --- | --- | --- | --- |
| (Street # & Name, City, Province, Postal Code) |  |  |  |
| E.g. 123 Street Ave, Toronto, ON, L5C 2B3 |  |  |  |

– End of Section –

Appendix B: Template for Measurement Data Control Sheet

This template is available in Online IESO.

| Resource ID | Contributor ID | Contributor Name | Commitment Month | Time Interval for which data was estimated |
| --- | --- | --- | --- | --- |

– End of Section –

References

| Document ID & Link | Document Title |
| --- | --- |
| MDP_RUL_0002 | Market Rules for the Ontario Electricity Market |
| IMP_GDE_0088 | Market Manual 1.3: Identity Management Operations Guide |
| PRO-408 | Market Manual 1.5: Market Registration Procedures |
| MDP_PRO_0027 | Market Manual 4.2: Submission of Dispatch Data in the Real-Time Energy and Operating Reserve Markets |
| IMP_PRO_0034 | Market Manual 4.3: Real-Time Scheduling of the Physical Markets |
| MDP_PRO_0045 | Market Manual 5.4: Prudential Support |
| MDP_PRO_0033 | Market Manual 5.5: Physical Markets Settlements Statements |
| IMP_PRO_0035 | Market Manual 7.3: Outage Management |
| IESO_MAN_0077 | Market Manual 9.2: Submitting Operational and Market Data for the DACP |

 End of Document 

Agreements between the relevant balancing authorities will, at a minimum set out requirements for firm transactions and confirmation that capacity transacted as a result of a capacity auction will be coordinated between the balancing authorities (e.g. removed from applicable adequacy assessments).

Online IESO is an online tool for market participants to submit data to the IESO; accessible at Online IESO (https://online.ieso.ca).

Median EFORd of Ontario’s natural gas fleet excluding Lennox Generating Station.

A time stamp refers to the time recorded by Online IESO when a capacity auction participant submits or revises an offer during the 2-business day offer submission window.

A residential customer refers to a smart-metered service account that is billed (by a licensed local distribution company) on a residential-rate class specified in a rate-order produced by the Ontario Energy Board. For the purposes of this program the term ‘residential’, as intended by the IESO, excludes ‘net-metered’ and/or ‘unit sub-metered’ customers.

Non-performance event means an event determined by the CMP where a demand response resource is, in whole or in part, in a Demand Response Outage or otherwise unable to Curtail for a period of time.

Submitted measurement data refers to the monthly Data submissions for the DR Resource in accordance with the Demand Response Submission Timelines.

Actual contributors’ measurement data refers to the individually submitted Contributor Data through the DR Audit task in Online IESO.

If a partial audit is conducted, the actual measurement data will only be assessed using Step 1 of the audit process.

For a capacity auction participant or capacity market participant that has not registered a resource in the energy market for a capacity obligation, the effective date of the buy-out request must be specified as the first day of the associated obligation period.