Market Manual 4: Market Operations

# Part 4.2: Operation of the Day-Ahead Market

Issue 4.1

June 3, 2026

This market manual provides information to market participants on the operation of the day-ahead market.

Document Change History

| Issue | Reason for Issue | Date |
| --- | --- | --- |
| 1.0 | Market Transition | November 11, 2024 |
| 2.0 | Issued in advance of MRP Go Live – May 1, 2025 | April 25, 2025 |
| 3.0 | Issue released for Baseline 54.0 | September 10, 2025 |
| 4.0 | Issue released for Baseline 54.1 | December 3, 2025 |
| 4.1 | Issue released for Baseline 55.1 | June 3, 2026 |

Related Documents

| Document ID | Document Title |
| --- | --- |
| MAN-109 | Market Manual 4.1: Submission of Dispatch Data in the Physical Markets |
| MAN-111 | Market Manual 4.3: Operation of the Real-Time Markets |

Table of Contents

Table of Contents	i

List of Figures	iii

List of Tables	iii

Table of Changes	iv

Market Manual Conventions	v

1	Introduction	1

1.1	Purpose	1

1.2	Scope	1

1.3	Contact Information	2

2	Operation of the Day-Ahead Market	3

2.1	Day-Ahead Market Calculation Engine	3

2.2	Day-Ahead Market Process Timeline	3

2.3	Day-Ahead Market Calculation Engine Initializing Conditions	4

2.3.1	Initial Hours of Operation	4

2.3.2	Treatment of Daily Dispatch Data over Midnight	5

2.3.3	Treatment of MGBRT over Midnight	5

2.3.4	Treatment of MGBDT over Midnight	6

2.3.5	Treatment of Thermal States for GOG-Eligible Resources	6

2.3.6	Treatment of Ramp Rates	7

2.3.7	Initial Schedules	7

2.3.8	Linked Forebays over Midnight	7

3	Day-Ahead Market Data Inputs	9

3.1	Market Participant Data	9

3.2	IESO Data Inputs	9

3.2.1	Constraint Violation Penalty Curves	9

3.2.2	Market Power Mitigation Information	9

3.2.3	IESO Reliability Requirements	9

3.2.4	Resource Reliability Constraints	10

3.2.5	Demand Forecasts	10

3.2.6	Centralized Variable Generation Forecast	11

3.2.7	IESO-Controlled Grid Information	11

3.2.8	Operating Reserve Requirements	11

4	Day-Ahead Market Scheduling Process	12

4.1	Day-Ahead Market Scheduling Process Execution	12

4.2	Rerun Authority	12

4.3	Delays to Day-Ahead Market Scheduling Process	13

4.4	Day-Ahead Market Scheduling Process Failure	13

5	IESO Day-Ahead Reliability Commitments for GOG-Eligible Resources	15

5.1	Principles for Applying Reliability Commitments	15

5.2	Process for Applying Reliability Commitments	16

6	Results from the Day-Ahead Market	17

6.1	Day-Ahead Market Resource Schedules	17

6.2	Day-Ahead Market Prices	17

6.3	Day-Ahead Operational Commitments	17

6.3.1	GOG-Eligible Resource Constraints for Combined Cycle Plants	18

6.4	Day-Ahead Market Boundary Entity Resource Schedules	19

6.5	Day-Ahead Market Economic Operating Points	19

7	Publishing and Issuing Day-Ahead Market Results	20

7.1	Day-Ahead Market Reports	20

7.2	DAM Notifications	26

7.3	Standby Notices and Reports for Hourly Demand Response Resources	26

8	Cancellation and Withdrawal of Day-Ahead Operational Commitments	28

8.1	Withdrawal from Commitment	28

8.2	IESO Cancellation of Day-Ahead Operational Commitments for GOG-Eligible Resources	28

8.3	Day-Ahead Operational Commitment Cancellation Cost Recovery	29

9	Day-Ahead Market Remediation	30

Appendix A: Day-Ahead Market Calculation Engine	31

A.1	Pass 1 – Market Commitment and Market Power Mitigation	31

A.2	Pass 2 – Reliability Scheduling and Commitment	32

A.3	Pass 3 – Day-Ahead Market Scheduling and Pricing	33

Appendix B: Detailed IHO Calculation	34

B.1	Last Status Change Time	35

B.2	Net Interchange Schedule Calculation	36

Appendix C: Constraint Violation Penalty Curves	37

Appendix D: Settlement Floor Price	38

List of Acronyms	39

References	40

List of Figures

Figure 2-1: Day-ahead Market Process Timeline	4

Figure 2-2: MGBRT Completion on the Next Dispatch Day	5

Figure 2-3: Managing Linked Forebays for the Start of the Day-ahead Market	8

Figure 6-1: Day-Ahead Schedule and Day-Ahead Operational Commitment	17

Figure 7-1: Day-Ahead Publishing and Reporting Processes	20

Figure A-1: Day-Ahead Market Calculation Engine Passes	31

List of Tables

Table 2-1: Satisfy MGBRT over Midnight	6

Table 7-1: Day-Ahead Market Public Report Descriptions	21

Table 7-2: Day-Ahead Market Confidential Report Descriptions	24

Table 7-3: Notifications – Day-Ahead Market	26

Table B-1: Last Status Change Time	35

Table of Changes

| Reference | Description of Change |
| --- | --- |
| Table 2-1 | - Update for Day-Ahead Market Initializing conditions for NQS resources |
| Appendix B | - Update for Day-Ahead Market Initializing conditions for NQS resources |

- Market Manuals

Market manuals set out procedural and administrative details with respect to market rule requirements. Where there is a conflict between the requirements described in a market manual or appended document, and those within the market rules, the market rules shall prevail.

Market Manual Conventions

The standard conventions followed for market manuals are as follows:

the word 'shall' denotes a mandatory requirement;

references to market rule sections and sub-sections may be appreviated in accordance with the following representative format: ‘MR Ch.1 ss.1.1-1.2’ (i.e. market rules, Chapter 1, sections 1.1 to 1.2);

references to market manual sections and sub-sections may be appreviated in accordance with the following representative format: ‘MM 1.5 ss.1.1-1.2’ (i.e. market manual 1.5, sections 1.1 to 1.2);

internal references to sections and sub-sections within this manual take the representative format: ‘sections 1.1 – 1.2’;

terms and acronyms used in this market manual in its appended documents that are italicized have the meanings ascribed thereto in MR Ch.11

data fields are identified in all capitals; and

references to “Day 0” mean the current day, references to “Day 1” mean the day immediately after the current day, references to “Day 2” mean the day two days after the current day, and so on.

– End of Section –

## Introduction

### Purpose

This market manual contains the information associated with the operation of the day-ahead market. It is intended to provide a summary of the steps and interfaces between market participants and the IESO during the operation of the day-ahead market.

The information in this market manual serves as a roadmap for market participants and the IESO, and reflect the requirements set out in the market rules and applicable IESO policies and standards.

### Scope

This market manual supplements the following market rules:

- MR Ch.4 s.7.3.5

- MR Ch.5 s.1.2.1

- MR Ch.5 s.3.2: Obligations of the IESO

- MR Ch.5 s.5.2.5

- MR Ch.5 s.6: Outage Coordination

- MR Ch.5 App.5.1: Performance Standards for Ancillary Services

- MR Ch.7 s.1.6.1.3

- MR Ch.7 s.2.3: Aggregated Generation Units, Electricity Storage Units or Sets of Load Equipment as Resources

- MR Ch,7 s.3.2.4: Submissions During the Day-Ahead Market Restricted Window

- MR Ch.7 s.3.2.5

- MR Ch.7 s.3.3.8: Obligation to Revise Dispatch Data

- MR Ch.7 s.3.3.9.1A

- MR Ch.7 s.3.4.1.8

- MR Ch.7 s.3.5: Energy Offers and Energy Bids

- MR Ch.7 s.3.12: Transmission System Information

- MR Ch.7 s.3A.1: Information Used by the IESO to Determine Schedules and Prices

- MR Ch.7 s.4.1: Day-Ahead Market Scheduling Process

- MR Ch.7 s.4.2: Determining the Day-Ahead Schedule

- MR Ch.7 s.4.3: Day-Ahead Market Scheduling Process Failure

- MR Ch.7 s.4.4: Administration of the Day-Ahead Market Calculation Engine

- MR Ch.7 s.4.5: Information Used by the Day-Ahead Market Calculation Engine

- MR Ch.7 s.4.6: Passes of the Day-Ahead Market Calculation Engine

- MR Ch.7 s.4.7: Publishing Day-Ahead Market Information

- MR Ch.7 s.4.8: Issuing Market Participant-Specific Day-Ahead Information

- MR Ch.7 s.5.2.2

- MR Ch.7 s.7.6: Dispatch Scheduling Errors

- MR Ch.7 s.8.4A: Administrative Pricing

- MR Ch.7 s.10.1.3

- MR Ch.7 s.10.3: Day-Ahead Operational Commitment and Pre-Dispatch Operational Commitment

- MR Ch.7 s.12.1.1.6

- MR Ch.7 s.12.1.3A

- MR Ch.7 s.19.4.2: Standby and Activation Notices

- MR Ch.7 s.22: Market Power Mitigation

- MR Ch.7 App.7.7: Radial Intertie Transactions

- MR Ch.9 s.3.3: Day-Ahead Market Balancing Credit

- MR Ch.9 s.4.11: 	Fuel Cost Compensation Credit

### Contact Information

Changes to this market manual are managed via the IESO Change Management process. Stakeholders are encouraged to participate in the evolution of this market manual via this process.

To contact the IESO, you can email IESO Customer Relations at customer.relations@ieso.ca or use telephone or mail. Telephone numbers and the mailing address can be found on the IESO website. IESO Customer Relations staff will respond as soon as possible.

– End of Section –

## Operation of the Day-Ahead Market

### Day-Ahead Market Calculation Engine

(MR Ch.7 s.4.6)

Day-ahead market calculation engine passes – The functions of each pass of the day-ahead market calculation engine are summarized in Appendix A.

### Day-Ahead Market Process Timeline

The timeline for the operation of the day-ahead market on the day before the relevant dispatch day is summarized as follows:

The IESO issues an adequacy report for the next dispatch day, at 05:30 EPT prusuant to MR Ch 7 s.12.1.1.6(a).

The day-ahead market submission window opens at 06:00 EPT. Standing dispatch data will be converted to dispatch data for the next day at 06:00 EPT (MR Ch.7 s.3.3.9.1A).

The submission deadline for requests for segregation for the day-ahead market time frame is:

08:00 EPT if an outage to critical equipment is required; or

09:00 EPT if no outage to critical equipment is required (MR Ch.7 App 7.7 s.1.3.3).

The day-ahead market regulation submission window closes at 09:00 EPT (MR Ch.5 App. 5.1 s.1.1).

The IESO issues the following reports for the next day at 09:00 EPT:

updated Adequacy Report for the next dispatch day (MR Ch.7 s.12.1.1.6(b))

Day-Ahead Intertie Scheduling Limit Report (MR Ch.7 s.4.7.1.1)

Day-Ahead Area Reserve Constraints Report (MR Ch.7 s.4.7.2.10)

The day-ahead market submission window closes at 10:00 EPT, marking the start of the day-ahead market restricted window and the start of the day-ahead market calculation engine run. Dispatch data may be accepted after 10:00 EPT only in accordance with MR Ch.7 s.3.2.4-3.2.5.

The IESO will typically issue day-ahead market results by 13:30 EPT. Upon the issuance of day-ahead market results, an updated Adequacy Report for the next day is issued (MR Ch.7 s.12.1.1.6C).

The IESO will issue day-ahead market results or declare a failure, by 15:30 EPT in accordance MR Ch.7 s.4.3.2. The day-ahead market restricted window concludes upon the issuance of day-ahead market results or the declaration of a failure.

The pre-dispatch calculation engine runs at 20:00 EST and includes any day-ahead operational commitments resulting from the day-ahead market. Figure 2-1 illustrates the interplay of the day-ahead market with the pre-dispatch process time frame

Figure 2-1: Day-ahead Market Process Timeline

### Day-Ahead Market Calculation Engine Initializing Conditions

(MR Ch.7 s.4.4.1)

Role of initializing conditions in day-ahead market participation – The day-ahead market calculation engine establishes initializing conditions of the IESO-administered markets and IESO-controlled grid for the next dispatch day. This section describes those conditions to allow market participants to manage dispatch data accordingly and to understand the results of the day-ahead calculation engine.

#### Initial Hours of Operation

Function of IHO – Initial Hours of Operation (IHO) refers to the number of consecutive hours a GOG-eligible resource is in operation at the end of the current dispatch day. The day-ahead market calculation engine uses the IHO to determine whether to process the start-up offer for the resource at the beginning of the next dispatch day (HE 01) and to facilitate the treatment of minimum generation block run-time (MGBRT) over midnight (HE 24).

Refer to Appendix B: Detailed IHO Calculation for details on how IHO is calculated.

#### Treatment of Daily Dispatch Data over Midnight

(MR Ch.7 s.3.5.12)

Start-up offer treatment – When the day-ahead market calculation engine determines the schedule for the first hour of the next day, it does not consider the start-up offers for GOG-eligible resources that are already in operation in the last hour of the current dispatch day as determined by the IHO. Table 2-1 describes the conditions under which start-up offers for HE 01 are considered by the day-ahead market calculation engine.

#### Treatment of MGBRT over Midnight

(MR Ch.7 s.10.3.4)

Operational Commitments over midnight – The day-ahead market calculation engine commits a generation resource at the beginning of the next day for a minimum number of hours required to satisfy the balance of its minimum generation block run time (MGBRT) from the previous day’s day-ahead operational commitment or pre-dispatch operational commitment (refer to Figure 2-2 below). It uses the MGBRT applicable to the dispatch day for which the day-ahead market calculation engine is being run to calculate the remaining MGBRT hours that carry over to the next dispatch day. If a resource is scheduled in HE24, but does not have a day-ahead operational commitment or pre-dispatch operational commitment, the day-ahead market calculation engine assumes that the resource’s MGBRT has already been satisfied.

Figure 2-2: MGBRT Completion on the Next Dispatch Day

Conditions for completing MGBRT on the next dispatch day – The day-ahead market completes the process of establishing any remaining MGBRT hours of a day-ahead operational commitment or pre-dispatch operational commitment, as described above, if it satisfies the following conditions:

it is a dispatchable generation resource that is a non-quick start resource and is not a nuclear generation resource;

it has valid offers for all the hours required to satisfy its remaining MGBRT in the next dispatch day; and

MGBRT – IHO ≥ 1

Table 2-1: Satisfy MGBRT over Midnight

| MLP Constraint Status HE 24 Day 0 | *Pre-dispatch Initial Schedule HE 24 Day 0 | Initial Hours of Operations (IHO) | Consider Start-up Offer HE 1 Day 1 | Satisfy MGBRT over Midnight |
| --- | --- | --- | --- | --- |
| YES = Constraint NO = No constraint | ≠0 = In operation 0 = Not in operation |  | YES = Start-up offer is considered NO = Start-up offer is not considered | YES = MGBRT is satisfied NO = MGBRT is not satisfied |
| YES | ≠0 | Min(PDIHO, CMIHO) | NO | YES |
| NO | ≠0 | Min(24, MGBRT) | NO | NO |
| NO | 0 | 0 | YES | NO |
| Input Data |  | Calculated Value | Treatment by Day-Ahead Market Calculation Engine |  |

* As determined by the most recent pre-dispatch schedule results for the current dispatch day prior to the initialization of the day-ahead calculation engine.

| CMIHO | = | The number of consecutive hours the dispatchable generation resource is constrained on in Resource Constraint Data at the end of Day 0 as determined by its active minimum constraint contracts Day 0 |
| --- | --- | --- |
| PDIHO | = | The number of consecutive hours the dispatchable generation resource has a schedule greater than zero at the end of Day 0 as determined by the most recent PD run for Day 0 |

#### Treatment of MGBDT over Midnight

Market participants must contact the IESO – The day-ahead market calculation engine does not respect a GOG-eligible resource’s minimum generation block down time (MGBDT) after midnight, which may result in a day-ahead operational commitment before the resource has satisfied its MGBDT. In the event that a GOG-eligible resource is committed without sufficient time to complete MGBDT, the market participant must contact the IESO.

#### Treatment of Thermal States for GOG-Eligible Resources

(MR Ch.7 s.3.5.35)

Initial thermal state – Registered market participants for GOG-eligible resources select one thermal state for use in the day-ahead market calculation engine applicable to ramp up energy to minimum loading point and start-up offer through the submission of the thermal state dispatch data parameter pursuant to MR Ch.7 s.3.5.35.

Thermal state for MGBDT – The day-ahead market calculation engine will use a hot thermal state for the MGBDT.

#### Treatment of Ramp Rates

(MR Ch.7 s.3.5.34)

Daily ramp rate – The day-ahead market calculation engine will establish day-ahead schedules using the daily ramp rate submitted for the next dispatch day in accordance with MR Ch.7 s.3.5.34.

#### Initial Schedules

Daily ramp rate over midnight – The day-ahead market calculation engine uses the most recent pre-dispatch schedule results for HE 24 for the current dispatch day to ensure that the day-ahead schedule respects the resource’s ramp rate submitted pursuant to MR Ch.7 s.3.5.34 for HE 01 of the next dispatch day.

#### Linked Forebays over Midnight

(MR Ch.7 s.3.5.23)

Dispatch data from previous dispatch day – The day-ahead market calculation engine uses the dispatch data parameters for establishing linked forebays as provided by MR Ch.7 s.3.5.23 for each dispatch day independently from the previous day irrespective of the submissions and schedules from the prior dispatch day. This may lead to infeasible day-ahead schedules, which may be managed by market participants as described below.

Resource evaluated independently – The day-ahead market calculation engine evaluates forebay-related dispatch data submitted on an upstream linked forebay and a downstream linked forebay independently from each other during the first h hours of the day-ahead market look-ahead period, where h is the value of the time lag submitted. Similarly, in the last h hours of the dispatch day, the upstream linked forebay will be independently evaluated of time lag and MWh ratio.

Dispatch data parameters – Market participants are expected to manage the risk of infeasible day-ahead schedules for resources with linked forebays over the midnight boundary. Registered market participants may consider submitting appropriate energy offer quantities, hourly must run quantities, minimum hourly output values or an outage slip.

Figure 2-3: Managing Linked Forebays for the Start of the Day-ahead Market

– End of Section –

## Day-Ahead Market Data Inputs

### Market Participant Data

(MR Ch.7 s.4.5.1)

Supplied by market participant – The day-ahead market calculation engine uses the following information supplied by market participants:

dispatch data;

regulation offers;

outage information including segregated mode of operation and planned demand control activities (e.g. transmitter or distributor voltage reductions, load disconnection); and

thermal ratings for the relevant portions of the transmission system (MR Ch.5 s.5.2.5)

### IESO Data Inputs

(MR Ch.7 s.4.5.1)

Supplied by IESO – The day-ahead market calculation engine considers the information supplied by the IESO, including information described in the following section. The IESO uses the most recent information available prior to the initiation of the day-ahead market calculation engine run.

#### Constraint Violation Penalty Curves

Overview – Constraint violation penalty curves are penalty functions used to prioritize the violation of constraints in the day-ahead market calculation engine (MR Ch.7 s.1.6.1.3). Refer to Appendix C for further detail.

#### Market Power Mitigation Information

Overview – The data in connection with the market power mitigation process is established in accordance with MR Ch.7 s.22.14.

#### IESO Reliability Requirements

Overview – Reliability requirements refer to reliability-related system constraints provided by the IESO including any system-wide and area-specific operating reserve requirements, security limits, maximum import and export limits, net interchange scheduling limit (NISL) and regulation capacity requirements. The IESO updates this information to reflect anticipated conditions for every dispatch hour.

#### Resource Reliability Constraints

(MR Ch.5 ss.1.2 and 3.2)

Manual constraints – The IESO may manually intervene and constrain resources to be scheduled to inject or withdraw energy at, above or below a specific value to maintain reliability. To ensure the resource is scheduled by the day-ahead market calculation engine, the IESO will create a scheduling constraint on the resource as an input to the day-ahead market calculation engine following notice to the affected market participant.

Timing of constraint – Resource constraints for reliability may be applied before or after the DAM expiration.

#### Demand Forecasts

(MR Ch.7 s.3A.1.7)

Demand forecast areas – The IESO produces average and peak hourly demand forecasts for the province as a sum of four separate demand forecasts for each of the following demand forecast areas:

Northeast: comprising of the Northeast electrical zone;

Northwest: comprising of the Northwest electrical zone;

Southeast: comprising of East, Essa, Ottawa, Toronto electrical zones;

Southwest: comprising of Bruce, Niagara, Southwest and West electrical zones.

The demand forecasts are generated based on historical demand levels as well as expectations of future demand.

Average demand forecast – The average non-dispatchable demand forecast is used in Pass 1 and Pass 3 of the day-ahead market calculation engine and represents the forecast demand quantities for all non-dispatchable loads and forecast losses in the demand forecast areas. The forecast quantities for demand for all non-dispatchable loads and losses are determined using the average demand forecasts, load distribution factors and bid quantities submitted for virtual hourly demand response resources and physical hourly demand response resources and self-scheduling storage resources registered to withdrawal.

Peak demand forecast – The peak non-dispatchable demand forecast is used in Pass 2 of the day-ahead market calculation engine and represents the forecast demand quantities for all non-dispatchable loads, price responsive loads, self-scheduling electricity storage resources and both dispatchable loads and dispatchable electricity storage resources where no bid is submitted, including forecast losses in the demand forecast areas. This forecast quantity is determined using the peak demand forecasts, load distribution factors and bid quantities submitted for virtual hourly demand response resources and physical hourly demand response resources.

#### Centralized Variable Generation Forecast

(MR Ch.4 s.7.3.5 and MR Ch.7 s.3A.1.6)

Pass 2 – The IESO variable generation centralized forecast is used in Pass 2 of the day-ahead market calculation engine.

Passes 1 and 3 – If submitted in accordance with MR Ch.7 s.3.5.18, the IESO will use the variable generation forecast quantity in Pass 1 and Pass 3 of the day-ahead market calculation engine. Otherwise, the IESO will use its variable generation centralized forecast in Pass 1 and Pass 3.

#### IESO-Controlled Grid Information

Overview – The IESO-Controlled grid information includes, but is not limited to:

power system model, which represents power flow relationships between locations on the IESO-controlled grid and between the IESO control area and neighbouring control areas;

the distribution of imports, exports, and any unscheduled flows (loop flows) between the integrated power system and neighbouring control areas or neighbouring transmission systems;

the status of power system equipment;

forced outages, which the IESO shall use reasonable efforts to incorporate, and approved planned outages in accordance with MR Ch.5 s.6;

load distribution factors;

a list of contingencies to be simulated;

a list of monitored equipment; and

the transmission system information provided by each transmitter pursuant to MR Ch.7 s.3.12.

#### Operating Reserve Requirements

(MR Ch.5 s.4.5.3)

Flexible operating reserve – The IESO determines the amount of operating reserve that is required by the system for any given hour. In addition to the minimum amount of operating reserve required to be maintained under the applicable reliability standards (MR Ch.5 s.4.5.2), the IESO may require additional thirty-minute operating reserve for flexibility pursuant to MR Ch.5 s.4.5.3.

– End of Section –

## Day-Ahead Market Scheduling Process

(MR Ch.7 s.4.7.2)

Timing – The day-ahead market scheduling process typically consists of one run of the day-ahead market calculation engine, beginning at 10:00 EPT and completing by 13:30 EPT.

### Day-Ahead Market Scheduling Process Execution

(MR Ch.7 ss.4.1 – 4.3)

Validation – Upon completion of the day-ahead market calculation engine, the IESO validates the results. If the results are valid, they are published or issued, as applicable, pursuant to MR Ch.7 s.4.7.2 or MR Ch.7 s.4.8.1, and final. If the IESO determines the results to be invalid, they are not published.

Invalid results – The IESO may determine results to be invalid, including for the purpose of MR Ch.7 s.4.3.1, for reasons that include:

the results include resources that are not required, or exclude resources that are required, because of incorrect inputs or calculations;

the day-ahead market calculation engine is unable to resolve two or more conflicting restrictions; or

an incorrect input causes a material change in pricing or schedules.

### Rerun Authority

(MR Ch.7 s.4.3.1)

Justification for rerun – The IESO may correct its inputs, such as outage information, centralized variable generation forecast quantities or pre-dispatch process results, and rerun the day-ahead market calculation engine pursuant to MR Ch.7 s. 4.3.1 to produce valid day-ahead market results only in the event of IESO errors or technical issues.

No rerun – The day-ahead market calculation engine is not rerun for changing system conditions. Any such changes will be considered in subsequent pre-dispatch and real-time evaluation processes. Following MR Ch.7 s.4.3.1, once day-ahead market results are published by the IESO, there are no further reruns of the day-ahead market calculation engine.

Publication by 15:30 EPT – The IESO will not exercise its rerun authority pursuant to MR Ch.7 s.4.3.1 unless it anticipates that the results can be published by 15:30 EPT. Refer to MR Ch.7 s.4.3.2.

DAM notification – The IESO will notify market participants of a rerun pursuant to MR Ch.7 s.4.3.1 in the form of a DAM notification.

### Delays to Day-Ahead Market Scheduling Process

(MR Ch.7 ss.4.7.2 and 4.8.1)

Timing of publication and issuance – In the event that issues arise during the execution of the day-ahead market scheduling process, the IESO will perform an assessment on whether the issue will cause a delay in meeting the 13:30 EPT publishing and issuance target.

DAM notification for delayed publication or issuance – If the IESO determines that the publishing or issuance target cannot be met, it will publish a DAM notification in accordance with MR Ch. 7 s.4.3.1. The DAM notification sets out the nature of the delay and an estimated time for results to be published. The IESO will use reasonable efforts to correct the issues, rerun the day-ahead calculation engine, and validate, publish and issue results by 15:30 EPT. A delay in the day-ahead market scheduling process also extends the day-ahead market restricted window until valid results are published.

### Day-Ahead Market Scheduling Process Failure

(MR Ch.7 ss.4.3.2 and 4.3.3)

Triggers for day-ahead market failure – The IESO will declare a day-ahead market failure in accordance with MR Ch.7 s.4.3.2 in the following circumstances:

the IESO expects that valid day-ahead market results will not be produced by 15:30 EPT; or

there is a failure or planned outage of the software, hardware or communications systems that prevents the IESO from publishing or issuing day-ahead market results.

DAM notification – The IESO will notify market participants about the failure of the day-ahead calculation engine in accordance with MR Ch.7 s.4.3.2, in the form of a DAM notification. In these circumstances, the DAM notification states that:

no day-ahead schedules or prices will be produced; and

no day-ahead operational commitments will be produced.

Day-ahead market restricted window – The day-ahead market restricted window will conclude upon declaration of a failure.

Market settlements – After the IESO has declared a day-ahead market failure, the dispatch day will be settled solely based on the market participant’s participation in the real-time market.

Approach to next-day scheduling – In the event of a day-ahead market failure, the IESO will conduct a reliability assessment based on the latest system conditions for the following dispatch day. If any control actions are required to ensure the secure and reliable operation of the IESO-controlled grid as a result of the reliability assessment, they will be communicated to the affected market participants prior to the first run of the pre-dispatch calculation engine at 20:00 EST. Following the reliability assessment, the IESO may issue reliability commitments as described in section 5.1.

Boundary entity resources – In the event of a failure of the day-ahead market, the IESO may facilitate scheduling on boundary entity resources in accordance with MR Ch.7 s.5.2.2.2.

– End of Section –

## IESO Day-Ahead Reliability Commitments for GOG-Eligible Resources

(MR Ch.5 s.1.2.1)

Reliability commitments in the day-ahead timeframe – Pursuant to MR Ch.5 s.1.2.1, the IESO may be required to issue reliability commitments for GOG-eligible resources in the day-ahead timeframe, including inputs into the day-ahead market calculation engine.

### Principles for Applying Reliability Commitments

(MR Ch.5 s.1.2.1 and Ch.7 s.10.1.3)

General principles – MR Ch.5 s.1.2.1 permits the IESO to implement a reliability commitment in the day-ahead timeframe only if intervention is necessary to ensure or maintain reliability. The IESO will give market mechanisms priority in scheduling resources; however, the IESO will intervene and commit a resource that is critical for meeting the next day’s reliability needs in accordance with the principles described below.

Least-cost evaluation – When more than one resource is available to satisfy the IESO’s reliability needs for the next dispatch day, the IESO will perform, to the extent possible, a least-cost evaluation to determine the resource(s) that should be committed.

Reliability commitments prior to 10:00 EPT – The IESO may implement reliability commitments prior 10:00 EPT where there is evidence suggesting that the market mechanisms will not schedule the resource during the day-ahead market or the pre-dispatch process. These commitments will be included as inputs into the day-ahead market calculation engine.

Reliability commitments after DAM expiration and before 20:00 EST – The IESO may implement reliability commitments after DAM expiration and before the first pre-dispatch calculation engine run at 20:00 EST when:

there is evidence suggesting that the market mechanisms will not schedule the resource in pre-dispatch or real time; and

it is necessary to issue a start-up notice prior to the pre-dispatch process pursuant to MR Ch.7 s.10.1.3.

### Process for Applying Reliability Commitments

(MR Ch.5 s.1.2.1)

Minimum constraints – The IESO will issue reliability commitments to a GOG-eligible resource by applying minimum constraints to ensure that the resource is scheduled to at least its submitted MLP and for at least its MGBRT. If the reliability commitment is implemented prior to 10:00 EPT, the constraints will be included in day-ahead schedules in addition to future pre-dispatch schedules. Alternatively, if implemented after DAM expiration, the constraints will only be included in future pre-dispatch schedules.

Market participant input – Before committing any resources, the IESO will contact the market participant to confer about any commitment actions the IESO is considering during the day-ahead timeframe.

– End of Section –

## Results from the Day-Ahead Market

(MR Ch.7 s.4.1)

### Day-Ahead Market Resource Schedules

(MR Ch.7 ss.4.1 and 4.8.1)

Eligible resources – The IESO issues a day-ahead schedule only for resources for which the registered market participant submitted dispatch data in the day-ahead market.

### Day-Ahead Market Prices

(MR Ch.7 ss.4.7.2.5 – 4.7.2.8)

Price calculations – Table 7-1 includes additional information related to day-ahead market prices.

### Day-Ahead Operational Commitments

(MR Ch.7 s.4.8.1.4)

Constraints – If a GOG-eligible resource receives a day-ahead operational commitment, the IESO will implement minimum constraints to the submitted MLP for the lesser of the hours where it received a day-ahead schedule or its MGBRT. For each separate start, the constraint will be applied to the beginning of the day-ahead schedule where the resource is scheduled at or above its submitted MLP.

Figure 6-1 provides an example of an applied constraint and the underlying day-ahead market schedule.

Figure 6-1: Day-Ahead Schedule and Day-Ahead Operational Commitment

Example – In the above example, a GOG-eligible generation resource has a day-ahead schedule that varies in MW quantity from 07:00 EST to 21:00 EST. Even though the entire day-ahead schedule will be financially binding, the day-ahead operational commitment will only be up to the resource’s submitted MLP and for the duration of the MGBRT. The day-ahead operational commitment is applied to the first available time associated with the day-ahead schedule, immediately after the resource ramps up.

Where withdrawal is necessary – Day-ahead operational commitments cannot be rejected by the market participant. If the resource cannot satisfy its day-ahead operational commitment, the associated market participant must complete the withdrawal process in accordance with MR Ch.7 s.10.3.2 or MR Ch.7 s.10.3.3, as applicable. Refer to section 8.1.

#### 6.3.1	GOG-Eligible Resource Constraints for Combined Cycle Plants

Combustion turbine (CT) – A generation resource associated with a CT for a combined cycle plant that is not aggregated pursuant to MR Ch.7 s.2.3 has a constraint applied based on its MLP with a constraint code of “DA-CMT”.

Steam turbine (ST) – A generation resource associated with a ST for a combined cycle plant that is not aggregated pursuant to MR Ch.7 s.2.3 has a constraint applied based on the number of CTs within the combined cycle plant that are committed in a given hour. If the ST does not have a day-ahead operational commitment for a given hour, then an ST constraint is not created since when not using the pseudo-unit model, the CT and ST must be committed independently. The constraints are described below.

For STs not using the pseudo-unit model:

The 1-on-1 ST MLP with a constraint code of “DA-CMT”, where one or zero associated CTs are scheduled to at or above their MLPs in a given hour, the ST received a commitment for that hour, and the ST is scheduled to at least the 1-on-1 ST MLP.

The n-on-1 ST MLP with a constraint code of “DA-CMT”, where n>1, there are n associated CTs scheduled to at or above their MLPs in a given hour, the ST received a commitment for that hour, and the ST is scheduled to at least the n-on-1 ST MLP.

If the ST received a day-ahead schedule that is less than the n-on-1 ST MLP, the ST constraint, for the applicable hours of the ST commitment, will be equal to the next lowest ST MLP for which it was economic with a constraint code of “DA-CMT”. There will be an additional constraint for these hours to the n-on-1 ST MLP with a constraint code of “COMCYC”.

For STs using the pseudo-unit model:

The n-on-1 ST MLP with a constraint code of “DA-CMT”, where n represents the number of CTs that are committed through an associated pseudo-units operating in combined cycle mode in a given hour.

### Day-Ahead Market Boundary Entity Resource Schedules

(MR Ch 7 s.5.2.2)

The day-ahead schedules for boundary entity resources will be used for the pre-dispatch calculation engine’s validation in accordance with MR Ch.7 s.5.2.2.

### Day-Ahead Market Economic Operating Points

Economic Operating Points – The IESO calculates the day-ahead lost cost economic operating points for resources. Refer to MR Ch.7 App.7.8 for more information.

– End of Section –

## Publishing and Issuing Day-Ahead Market Results

This section describes the reports the IESO publishes and issues relating to the day-ahead market.

Figure 7-1: Day-Ahead Publishing and Reporting Processes

### Day-Ahead Market Reports

(MR Ch.7 ss.4.7 and 4.8)

Table 7-1 provides a list and description of each day-ahead market report that is published by the IESO in accordance with the applicable section of the market rules. Public reports are available to all market participants and to the broader public.

Table 7-1: Day-Ahead Market Public Report Descriptions

| Report Name | Report Description |
| --- | --- |
| Adequacy Report (MR Ch.7 s.12.1.1.6) | The Adequacy Report: To support the day-ahead market, is published in accordance with MR Ch.7 ss.12.1.1.6(a), 12.1.1.6(b), 12.1.1.6(c); refer to MM 7.2 s.3.1.1 for more information; and presents information with hourly granularity. |
| Day-Ahead Area Operating Reserve Shortfall Report (MR Ch.7 s.4.7.2.1) | The Day-Ahead Area Operating Reserve Shortfall Report: contains operating reserve included in day-ahead schedule and any shortfalls in each hour, by area; and presents information with hourly granularity. |
| (Pre-Day-Ahead Market) Day-Ahead Area Reserve Constraints Report (MR Ch.7 s.4.7.1.2) | The (Pre-Day-Ahead Market) Day-Ahead Area Reserve Constraints Report: contains expected hourly maximum and minimum constraints for the area operating reserve regions expected to be used as inputs for the day-ahead market calculation engine; is typically published at approximately 09:00 EPT; and presents information with hourly granularity. |
| Day-Ahead Area Reserve Constraints Report | The Day-Ahead Area Reserve Constraints Report: contains hourly maximum and minimum constraints for the area operating reserve regions used as inputs for the day-ahead market calculation engine; presents information with hourly granularity; and is typically published at approximately 13:30 EPT |
| Day-Ahead Totals Report (MR Ch.7 s.4.7.2.2) | The Day-Ahead Totals Report: contains forecasts and schedules of system-wide information; and presents information with hourly granularity. |
| Pre-DAM Intertie Scheduling Limits Report (MR Ch.7 s.4.7.1.1) | The Pre-DAM Intertie Scheduling Limits Report: contains expected intertie scheduling limits for each intertie zone to be used as inputs to the day-ahead market calculation engine run; is typically published at approximately 09:00 EPT; and presents information with hourly granularity. |
| Day-Ahead Intertie Scheduling Limits Report (MR Ch.7 s.4.7.1.1) | The Day-Ahead Intertie Scheduling Limits Report: contains intertie scheduling limits for each intertie zone used as inputs to the day-ahead calculation engine; presents information with hourly granularity; and is typically published at approximately 13:30 EPT |
| Day-Ahead Security Constraints Report (MR Ch.7 s.4.7.2.3) | The Day-Ahead Security Constraints Report: contains binding security constraints applicable to the transmission system, as determined by the day-ahead market calculation engine; and presents information with hourly granularity. |
| Day-Ahead Virtual Transactions Report (MR Ch.7 s.4.7.2.4) | The Day-Ahead Virtual Transactions Report: contains aggregated sums of energy offers and bids submitted for each virtual transaction zone; contains aggregated sums of cleared virtual transactions for each virtual zonal resource; and presents information with hourly granularity. |
| Day-Ahead Hourly Energy LMP Report (MR Ch.7 s.4.7.2.5) | The Day-Ahead Hourly Energy LMP Report: contains the locational marginal pricing information in respect of energy for every delivery point, including the Energy Congestion Price and Energy Loss Price; and presents information with hourly granularity. |
| Day-Ahead Hourly Virtual Zonal Energy Price Report (MR Ch.7 s.4.7.2.6) | The Day-Ahead Hourly Virtual Zonal Energy Price Report: contains the virtual zonal price for each virtual zonal transaction zone; and presents information with hourly granularity. |
| Day-Ahead Hourly Ontario Zonal Energy Price Report (MR Ch.7 s.4.7.2.7) | The Day-Ahead Hourly Ontario Zonal Energy Price Report: contains the Ontario zonal price for the day-ahead market; and presents information with hourly granularity. |
| Day-Ahead Hourly Operating Reserve LMP Report (MR Ch.7 s.4.7.2.5) | The Day-Ahead Hourly Operating Reserve LMP Report: contains the locational marginal pricing-related information in respect of each class of operating reserve for every delivery point, including the Operating Reserve Congestion Price; and presents information with hourly granularity. |
| Day-Ahead Hourly Intertie Energy LMP Report (MR Ch.7 s.4.7.2.8) | The Day-Ahead Hourly Intertie Energy LMP Report: contains locational marginal price information for intertie zones in respect of energy, including internal congestion, losses, congestion due to intertie limits and congestion due to NISL constraints; and presents information with hourly granularity. |
| Day-Ahead Hourly Intertie Operating Reserve LMP Report (MR Ch.7 s.4.7.2.8) | The Day-Ahead Hourly Intertie Operating Reserve LMP Report: contains intertie zone locational marginal prices in respect of the non-spinning ten-minute operating reserve and thirty-minute operating reserve, internal congestion, congestion due to intertie limits and congestion due to NISL constraints; and presents information with hourly granularity. |
| Variable Generation Forecast Summary Report | The Variable Generation Forecast Summary Report: contains regional energy forecast for the next 48 hours, by fuel type, for all variable generation resources subject to centralized forecasting; is typically published approximately five minutes prior to every hour; presents information with hourly granularity. |
| Variable Generation Tie Breaking Rankings Report (MR Ch.7 s.4.7.4) | The Variable Generation Tie Breaking Rankings Report: contains variable generation tie-breaking rankings for the 90-day period; is typically published on the 1st calendar day of every month; is typically published if the tie-breaking ranking is updated to account for newly registered variable generation resources coming into service; and presents information with daily granularity. |
| Day-Ahead Constraints Shadow Prices Report (MR Ch.7 s.4.7.4) | The Day-Ahead Constraints Shadow Prices Report: contains shadow prices for the binding security constraints applicable to the transmission system, as determined by the day-ahead market calculation engine. contains information from five days before the date of publication; is typically published at approximately 08:00 EST; and presents information with hourly granularity. |
| Planned Transmission Outages Occurring Next 30 Days (MR Ch.5 s.6.5.2) | The report Planned Transmission Outages Occurring Next 30 Days: contains planned transmission outages that are requested by transmitters for the next 30 days; contains planned starting dates and end dates for transmission outages; and is typically published daily at approximately 17:00 EST |
| Day-Ahead Global Market Power Conditions for Energy Report (MR Ch.7 s.4.7.5) | The Day-Ahead Global Market Power Conditions for Energy Report: contains a summary of the hours in the study period when the price and import restriction conditions in respect of energy are met in the day-ahead market; and presents information with hourly granularity. |

Confidential reports –Table 7-2 provides a list and description of each day-ahead market confidential report issued by the IESO. Confidential reports are available only to the market participant to which the information relates.

Table 7-2: Day-Ahead Market Confidential Report Descriptions

| Report / Display Name | Report Description |
| --- | --- |
| Dispatch Data Report for the Day-Ahead Scheduling Process (MR Ch.7 s.4.8.1.1 and 4.8.1.7) | The Dispatch Data Report for Day-Ahead Scheduling Process: contains a summary of the dispatch data for energy submitted for the day-ahead market for each of the market participant’s resources, including the quantities that define the availability declaration envelope; is typically issued at approximately 13:30 EPT; and presents information with hourly and daily granularity, respectively. |
| Day-Ahead Market Operating Reserve Offer Report | The Day-Ahead Market Operating Reserve Offer Report: contains the offers for operating reserve used by the day-ahead market calculation engine; is typically issued at approximately 13:30 EPT; and presents information with hourly granularity. |
| Day-Ahead Pseudo-Unit Computed Values Report (MR Ch.7 s.4.8.1.2) | The Day-Ahead Pseudo-Unit Computed Values Report: contains the values used by the day-ahead market calculation engine for pseudo-units and generation resources associated with the corresponding combustion turbine generation units and steam turbine generation units; is based on market participant submitted registration and dispatch data for generation resources associated with the corresponding combustion turbine generation units and steam turbine generation units, and outages and constraints; and presents information with hourly granularity. |
| Day-Ahead Schedule Report (MR Ch.7 ss.4.8.1.3 and 4.8.1.8) | The Day-Ahead Schedule Report: contains day-ahead schedules for energy and operating reserve; notifies market participants that they have failed the conduct and impact test for price impact, if applicable; for combined cycle plants with pseudo-units, contains schedules for pseudo-units and for the corresponding resources for the combustion turbine and steam turbines generation units; and presents information with hourly granularity. |
| Day-Ahead Commitments Report (MR Ch.7 s.4.8.1.4) | The Day-Ahead Commitments Report: Includes day-ahead operational commitments and commitments implemented to maintain reliability; and presents information with hourly granularity. |
| Variable Generation Forecast by Resource Report (MR Ch.4 s.7.3.5) | The Variable Generation Forecast by Resource Report: contains an hourly energy forecast for each of the variable generator’s variable generation resources for the next 48 hours; is typically issued approximately five minutes prior to every hour; and presents information with hourly granularity. |
| Demand Response Standby Report (MR Ch.7 s.19.4.2) | The Demand Response Standby Report: notifies the capacity participants when their resources are on standby for demand response activations; if applicable, is issued after the day-ahead market calculation engine or pre-dispatch calculation engine produces valid results in respect of a business day; may be issued until 07:00 EST of the relevant dispatch day; and if a capacity market participant will not be placed on standby for the relevant dispatch day, it will receive confirmation of same at approximately 07:00 EST. |

### DAM Notifications

(MR Ch.7 s.12.1.3A)

Website interface – Market participants can receive DAM notifications by accessing “DAM Notifications” on the IESO’s website or:

by logging onto the Energy Market Interface; or

by using the Application Programmers Interface.

Table 7-3 contains notices published pursuant to MR Ch.7 ss.4, 8.4A and 7.6 that pertain to the day-ahead market.

Table 7-3: Notifications – Day-Ahead Market

| Notification | Description |
| --- | --- |
| DAM Notification: Additional Dispatch Data | - The IESO is approving revisions to dispatch data during the day-ahead market restricted window for all market participants. |
| DAM Notification: Delay in Initialization of Day-Ahead Market Calculation Engine | - The IESO delays the initialization of the day-ahead market calculation engine. The IESO inputs into the day-ahead market calculation engine will reflect the information available at the initialization, which may have changed since 10:00 EPT. |
| DAM Notification: Delay to Publication of Day-Ahead Market Results | - The IESO delays the publication of day-ahead market results later than 13:30 EPT. An expected time of publication may be indicated, if available. |
| DAM Notification: DAM Failure | - The IESO declares a day-ahead market failure for the affected dispatch day. The IESO will indicate any changes to scheduling of boundary entity resources in accordance with MR Ch.7 s.5.2.2.2. |
| Administrative Pricing Notification | - The IESO establishes administrative prices for the day-ahead market in accordance with MR Ch.7 s.8.4A. |
| Dispatch Scheduling Error Notification | - The IESO declares a dispatch scheduling error with respect to the day-ahead market results in accordance with MR Ch.7 s.7.6.1.2. |

### Standby Notices and Reports for Hourly Demand Response Resources

(MR Ch.7 s.19.4.2)

Standby reports – After the successful completion of the day-ahead market calculation engine run, the IESO may issue a Standby Notice in accordance with MR Ch.7 s.19.4.2. Hourly demand response resources that did not receive a standby notice resulting from the day-ahead market are assessed during the pre-dispatch process to determine if they are required to be on standby.

– End of Section –

## Cancellation and Withdrawal of Day-Ahead Operational Commitments

(MR Ch.7 s.10.3)

### 8.1	Withdrawal from Commitment

(MR Ch.7 ss.3.3.8, 10.3.2 and 10.3.3)

Form of notice – For the purpose of providing notice to the IESO under MR Ch.7 s.10.3.2 or 10.3.3, a market participant must call the IESO prior to removing or revising its real-time market offers.

Revise dispatch data – If a market participant for a GOG-eligible resource expects not to satisfy a day-ahead operational commitment in accordance with MR Ch.7 s.10.3.2 or 10.3.3., it must revise the applicable dispatch data in accordance with MR Ch.7 s.3.3.8.

### 8.2	IESO Cancellation of Day-Ahead Operational Commitments for GOG-Eligible Resources

(MR Ch.7 ss.3.3.8 and 10.3.1)

Reasons for IESO cancelling commitment – The IESO will only cancel a day-ahead operational commitment under MR Ch.7 s.10.3.1 if doing so is necessary to maintain reliability, and not for economic reasons.

Form of notice – For the purpose of providing notice to market participants under MR Ch.7 s.10.3.1, the IESO will call market participants before cancelling the day-ahead operational commitment.

Revise dispatch data – Where a market participant for GOG-eligible resource will not be supplying committed energy as a result of the IESO cancelling a day-ahead operational commitment in accordance with MR Ch.7 s.10.3.1, the registered market participant must revise the applicable dispatch data in accordance with MR Ch.7 s.3.3.8.

### 8.3	Day-Ahead Operational Commitment Cancellation Cost Recovery

(MR Ch.9 ss.3.3 and 4.11)

Cancellation after the start of a day-ahead operational commitment – In the event that the IESO cancels a day-ahead operational commitment after the resource has reached its MLP, the total start-up offer will be included in the assessment of the generator offer guarantee as well as the speed no-load offer incurred for the hours that the resource was operating at or above MLP.

Cancellation before the start of a day-ahead operational commitment – If the IESO cancels a day-ahead operational commitment before the resource has reached its MLP, the start-up offer will not be included in the assessment of the generator offer guarantee. The IESO may compensate market participants for the cost incurred in securing unused fuel. Refer to MM 5.5 s.2.25 for further information regarding the fuel cost compensation credit.

Potential compensation for cancelled commitments – The resource may be eligible for the day-ahead market balancing credit, refer to MM 5.5 s.2.9.

– End of Section –

## Day-Ahead Market Remediation

(MR Ch.7 ss.7.6 and 8.4A)

No DAM failure – Market remediation may be conducted if the IESO identifies an error after the day-ahead market results are published or issued that was caused by an IESO input error. This is only applicable where the IESO has not declared a failure of the day-ahead market.

Retroactive administrative pricing – The IESO will retroactively establish day-ahead market administrative prices provided the requirements under MR Ch.7 s.8.4A are satisfied, including, but not limited to, the requirement to be administered within four business days after the affected dispatch day pursuant to MR Ch.7 s.8.4A.2.

Scope of administrative pricing – In response to an eligible error, the IESO may establish administrative prices for one or more locational marginal prices.

Dispatch Scheduling error – Further to MR Ch.7 s.7.6.1.2, the IESO will declare a dispatch scheduling error in the day-ahead market where:

the IESO identifies a pricing error in the day-ahead market which is identified within four business days after the dispatch day and cannot be corrected by normal administration methods in accordance with MR Ch.7 ss.8.4A.2 and 8.4A.3;

the IESO does not identify a pricing error that has occured in the day-ahead market within four business days after the dispatch day (MR Ch.7 s.8.4A.3); and

the IESO identifies an error that has impacted day-ahead schedules, regardless of how many days have elapsed since the relevant dispatch day.

Administrative pricing notification – The IESO will publish an administrative pricing notification in accordance with MR Ch.7 s.8.4A.3A in the following circumstance:

the IESO has established day-ahead market administrative prices in accordance with MR Ch.7 s.8.4A.

Dispatch scheduling error notification – The IESO will publish a dispatch scheduling error notification in the following circumstance:

a dispatch scheduling error has occurred in accordance with MR Ch.7 s.7.6.1.

– End of Section –

## Appendix A: Day-Ahead Market Calculation Engine

Day-ahead market calculation engine passes – Figure A-1 summarizes the functions of each pass of the day-ahead calculation engine.

Figure A-1: Day-Ahead Market Calculation Engine Passes

### A.1	Pass 1 – Market Commitment and Market Power Mitigation

(MR Ch.7 s.4.6.1.1)

Co-optimization – The Market Commitment and Market Power Mitigation pass (Pass 1) co-optimizes energy and operating reserve for the next dispatch day to meet the IESO’s hourly zonal average non-dispatchable demand forecasts and hourly operating reserve requirement. Pass 1 determines a set of initial prices, resource schedules and, for GOG-eligible resources, day-ahead operational commitments.

Evaluation – The data evaluated in Pass 1 includes non-dispatchable generation resource schedules and dispatch data for dispatchable generation resources or electricity storage resources, self-scheduling generation resources, dispatchable loads, price responsive loads, hourly demand response resources, imports, exports and virtual transactions.

Ex-ante market power mitigation – Pass 1 will perform the tests related to the ex-ante market power mitigation process provided by MR Ch.7 ss.22.13 and 22.14.

### A.2	Pass 2 – Reliability Scheduling and Commitment

(MR Ch.7 s.4.6.1.2)

Reliability scheduling – The Reliability Scheduling and Commitment pass (Pass 2) assesses whether additional resources are required to meet hourly zonal peak non-dispatchable demand and hourly operating reserve requirements.

Virtual transactions – Pass 2 does not evaluate virtual transactions.

Price responsive loads/variable generation resources – Pass 2 will use the IESO centralized variable generation forecast quantities provided by MR Ch.4 s.7.3.5 and the IESO’s forecast of demand from price responsive loads and self-scheduling electricity storage resources intending to withdraw. Pass 2 will use this information in lieu of bids submitted for price responsive loads and self scheduling electricity storage resources, and submitted variable generation forecast quantities.

Day-ahead operational commitments – Pass 2 may establish any day-ahead operational commitments in addition to those established in Pass 1.

Boundary entity resource schedules – Import schedules for boundary entity resources will not decrease and export schedules for boundary entity resources will not increase from Pass 1.

Information carried over from Pass 1 – Pass 2 will use the following information from Pass 1:

provisional day-ahead schedules and day-ahead operational commitments from Pass 1; and

dispatch data used in Pass 1, including any revisionsin accordance with the applicable market power mitigation processes provided by MR Ch.7 s.22.

### A.3	Pass 3 – Day-Ahead Market Scheduling and Pricing

(MR Ch.7 s.4.6.1.3)

Financially-binding outcomes – The Day-Ahead Market Scheduling and Pricing pass (Pass 3) produces the financially binding day-ahead schedules and prices, and day-ahead operational commitments.

Day-ahead operational commitments – Pass 3 may establish any day-ahead operational commitments in addition to those established in Passes 1 and 2.

Boundary entity resource schedules – Import schedules for boundary entity resources will not decrease and export schedules for boundary entity resources will not increase from Pass 2.

Information carried over from Pass 1 – Pass 3 uses the same set of market participant and IESO inputs used in Pass 1, including any revisions in accordance with the applicable market power mitigation processes provided by MR Ch.7 s.22.

– End of Appendix –

## Appendix B: Detailed IHO Calculation

Initial hour of operation (IHO) is used to process start-up offers for generators for input to the day-ahead market calculation engine and facilitate the treatment of MGBRT over midnight. The day-ahead market calculation engine will not consider start-up offers for dispatchable generation resources that are already in operation in the last hour of the current day to determine the first hour of the day-ahead schedule. The day-ahead market calculation engine will determine the number of hours the generation resource must run to satisfy any MGBRT requirement remaining from the previous day’s commitment.

Determining IHO will be triggered by the calculation of resource initial schedule (RIS). RIS is computed as part of each day-ahead market calculation engine run, and represents the dispatchable resource’s schedule in HE24 as determined by the most recent pre-dispatch schedule results for the current dispatch day. RIS is determined only for dispatchable generation resources and loads.

The calculation of IHO will use:

the results of the most recent pre-dispatch calculation engine results for Day 0; and

the Day 0 ‘constrained on’ status in the Contract Manager application from the most recent pre-dispatch calculation engine results for Day 0.

For pseudo-units, this determination is based on the combustion turbine (CT) associated with the pseudo-unit, not the ST.

For the nth resource IHO is determined as follows:

IHOn=0

IHOn=0,  &if RSIn=0 min24, MGBRTn,  &if RSIn≠0 and CMCS24n="NO"minPDIHOn,CMIHOn,  &otherwise   (1)

IHOn=min(PDIHOn, CMIHOn)

Where:

| IHOn | = | A non-negative integer representing the consecutive hours of operation of a resource before the end of the current day (Day 0) |
| --- | --- | --- |
| RISn | = | Dispatchable generation resource n initial resource schedule |
| CMCS24n | = | Dispatchable generation resource n constrained on status in the last study time of Day 0 as determined by whether it has an active minimum constraint contract in the last study time of Day 0 in Resource Constraint Data: Yes – denotes constrained on with minimum constraint contracts No – denotes no constraint |
| CMIHOn | = | The number of consecutive hours the dispatchable generation resource n is constrained on in Resource Constraint Data at the end of Day 0 as determined by its active minimum constraint contracts Day 0 |
| PDIHOn | = | The number of consecutive hours the dispatchable generation resource n has a schedule greater than zero at the end of Day 0 as determined by the most recent PD run for Day 0 |

Based on the above formula, the IHO can have the following values:

| IHO = 0 | The dispatchable generation resource is not in operation in the last study time of Day 0 (i.e. RIS = 0) |
| --- | --- |
| IHO = min(24, MGBRT) | The dispatchable generator is in operation in the last study time of Day 0 (i.e., RIS ≠ 0); and The dispatchable generator does not have a constraint in the last study time of Day 0 in Resource Constraint Data as determined by active MIN contracts in Day 0. |
| IHO = min(PDIHO, CMIHO) | The dispatchable generator is in operation in the last study time of Day 0 (i.e., RIS ≠ 0); and The dispatchable generator has a constraint in the last study time of Day 0 in Resource Constraint Data as determined by active MIN contracts in Day 0. |

### B.1	Last Status Change Time

The Last Status Change Time represents the time when the last status change occurred. The rules for calculating the Last Status Change Time are listed in Table B-1.

Table B-1: Last Status Change Time

| Initial Status | Initial Schedule (RIS) | IHO | CMCS24 | Last Status Change Time (YYYYMMDD HH:MM:00) |
| --- | --- | --- | --- | --- |
| OFF | 0 | 0 | No | The timestamp of the last hour of Day 0 – MGBDT hours This is to ensure that the generation resource will not be kept off at the beginning of the day due to a MGBDT constraint. |
| ON | ≠ 0 | Min(PDIHO, CMIHO) | Yes | The timestamp of the first hour of Day 1 – IHO hours |
| ON | ≠ 0 | Min(24, MGBRT) | No | The timestamp of the first hour of Day 1 – min(24, MGBRT) hours. This is to ensure that the generation resource will not be kept on at the beginning of the day due to a MGBRT constraint. |

Note: To satisfy the generation resource MGBRT over midnight, the day-ahead market calculation engine uses IHO and Day 1 MGBRT (not Day 0 MGBRT). A dispatchable generation resource may receive a day-ahead market schedule at the end of a day prior to the dispatch day, even if MGBRT has not been completed within that day. On the next day prior to the dispatch day (Day 1), the day-ahead market calculation engine will commit the dispatchable generation resource at the beginning of the day to satisfy its incomplete MGBRT from the previous day (Day 0).

### B.2	Net Interchange Schedule Calculation

The initial net interchange schedule is calculated as Total Imports minus Total Exports, where:

Total imports = ∑ Energy imports from all external sources for the last study time from the most recent pre-dispatch run

Total exports = ∑ Energy exports to all external sinks for the last study time from the most recent pre-dispatch run

– End of Appendix –

## Appendix C: Constraint Violation Penalty Curves

See Appendix A of Market Manual 4.3: Operation of the Real-Time Markets, for information on Constraint Violation Penalty Curves used by the calculation engines.

– End of Appendix –

## Appendix D: Settlement Floor Price

(MR Ch.7 s.1.6.1.4)

Please refer to Appendix C in Market Manual 4.3: Operation of the Real-Time Markets, for information on the settlement floor price.

– End of Appendix –

- List of Acronyms

| Acronym | Term |
| --- | --- |
| ADE | availability declaration envelope |
| CRS | commercial reconciliation system |
| CT | combustion turbine |
| DAM | day-ahead market |
| DSO | dispatch scheduling and optimization |
| EPT | Eastern Prevailing Time |
| EST | Eastern Standard Time |
| GOG | generator offer guarantee |
| IHO | initial hour of operation |
| MGBDT | minimum generation block down-time |
| MGBRT | minimum generation block run-time |
| MIN | minimum |
| MLP | minimum loading point |
| MMCP | maximum market clearing price |
| MW | megawatt |
| MWh | megawatt hour |
| NISL | net interchange scheduling limit |
| RIS | resource initial schedule |
| SMO | segregated mode of operation |
| ST | steam turbine |
| VG | variable generation |

– End of Section –

- References

| Document ID | Document Title |
| --- | --- |
| RUL-6 to RUL-24 | Market Rules |
| MAN-116 | Market Manual 5.5: IESO-administered Markets Settlement Amounts |
| MAN-121 | Market Manual 7.1: IESO-controlled Operating Procedures |
| MAN-122 | Market Manual 7.2: Near-Term Assessments and Reports |

– End of Document –